Biomedical Signal Processing and Control

A Fully Automatic Model for Premature Ventricular Heartbeat

Arrhythmia Classification using the Internet of Medical Things

Qurat-ul-ain Mastoi1, Asadullah Shaikh2,∗, Mana Saleh Al Reshan2 Adel Sulaiman2, M.A. Elmagzoub2,*, and Sultan AlYami2

1Department of Computer Science and Creative Technologies, University of the West England, Bristol, BS16QY, United Kingdom

2College of Computer Science and Information Systems, Najran University, Najran 61441, Saudi Arabia

Email addresses: Qurat-Ul-Ain.Mastoi@uwe.ac.uk (Qurat-ul-ain Mastoi),
asshaikh@nu.edu.sa (Asadullah Shaikh),
msalreshan@nu.edu.Sa (Mana Saleh Al Reshan),
aaalsulaiman@nu.edu.sa (Adel Sulaiman)
 meabdullah@nu.edu.sa (M.A. Elmagzoub)
 ssalamer@nu.edu.sa (Sultan AlYami)

Abstract

Cardiac arrhythmias are one of the leading causes of increased mortality worldwide and place a heavy burden on the medical environment. Premature ventricular contraction is the disturbance in electrical activity which is the most dangerous arrhythmia. Frequent occurrence of this type of arrhythmia in a regular heartbeat can lead to sudden cardiac death. Over the last decades, contemporary health-related device usage has increased the demand for efficient computational models for real-time analysis of cardiac arrhythmia. Despite notable experiments that have been done in the past decades, due to the intricate nature of PVC arrhythmia, success stories are still unsatisfying. There are numerous morphological and temporal variations present in ECG signals due to the inter-patient variability issue; extracting important characteristics of ECG signals is the most challenging task. As a result, there is a need to investigate the exact features of PVC arrhythmia, which assist in avoiding biased diagnosis. Precisely predicting it is a difficult task due to the negative polarity of PVC arrhythmia, the irregular mechanic of the ECG cycle, and anomalies between the normal cardiac rhythm. Furthermore, most of the studies in the literature followed the public benchmark dataset for the PVC arrhythmia classification, which is already pre-processed dataset. This study opens the door for a new direction of research using our unique, fully automatic model for PVC arrhythmia classification (FAPAC). This study designed an ECG monitoring module using the IoMT devices to obtain the real-time dataset for experiments and extract the relevant features from ECG signals. To classify the ECG beats, the fastest extended version of the recurrent neural network (RNN) model cyclic echo state networks to predict PVC arrhythmia. Our proposed FAPAC model successfully achieved 99.97% of accuracy, 99.99 % sensitivity,99.99% specificity, and 99.98% positive predictivity using the MIT-BIH-arrhythmia dataset, which is relatively higher than compared studies.

Keywords: Feature extraction, real-time, Internet of Medical Things, cardiac disease

Introduction

With a steady increase in the human population and spending on medical growth, it has become one of the general critical problems for each folk and government. The increasing mortality ratio increases researchers' attention to the automated classification of ventricular arrhythmia. As ECG tool is considered a primary non-invasive technique for a medical practitioner to analyze the condition of the heart. This research aims to design an ECG monitoring module using Internet of Medical Things components for the ECG acquisition for real-time analysis of arrhythmia. The major concept of this study is to design an algorithm that identifies the critical and non-critical heartbeat automatically to reduce the ratio of mortality. In previous studies, most of the researchers used the public benchmark dataset to design the algorithm which is ineffective because the real-time collection of signals is different from that of already preprocessed and stored datasets. To analyze the real-time condition of the patient, this research contributes to the design ECG monitoring module and modified Pan and Tompkins algorithm which satisfies the requirement of accurate detection of arrhythmia and critical heartbeat which was not considered previously. In the real-time collection of ECG signals, we have observed that it retains many different types of noisy information and variation which requires heavy pre-processing for efficient analysis [1][2], for instance, negative polarity of QRS and missing waves. In recent years, studies used time-domain analysis [3][4], frequency-domain analysis [5][6], and ECG signal analysis using wavelet transform [7]–[9] for the aim of analyzing the disorder in the heart or identify the heart abnormalities in a heartbeat. Researchers are working hard to develop a computer-assisted diagnostic method for PVC arrhythmia classification. Still, most of the studies are limited to extracting the general type of arrhythmia and used public benchmark datasets for their experiments. In literature, Bayesian classification techniques for premature ventricular contraction arrhythmia developed by [26] the authors claim that their proposed algorithm is generative and straightforward due to the probability distribution. According to the author's assumptions, their study needs to improve to extract more accurate features for the classification of PVC beat. [27] proposed evolutionary optimization to conduct a new experiment for feature extraction. A total of six features were selected to feed the SVM classifier to identify the PVC beat and normal beat. RF classifiers are proposed by [28] to determine the PVC arrhythmia from the ECG signals. This study only selects three PVC detection attributes: RR interval, QRS complex, and R amplitude. [1] proposed the improved template matching technique to identify PVC arrhythmia in ECG signals. The most significant parts of ECG morphology are the R-peak and QRS complex [29], [30]. Pan and Tompkins's algorithm plays a benchmark role to detect the QRS complex and R-peak intervals. [31] used the pan and Tompkins (PT) algorithm to extract R peaks from the de-noised ECG signal. The proposed technique detects the R-peak and QRS complex from ECG signals efficiently using less time. However, the limitation is observed in that studies only focus on regular peak intervals. [29] used Pan and Tompkins algorithm to extract R-peaks from the ECG signals. This method has the potential to provide excellent detection performance. However, the authors determined in the study that improvements are still needed to obtain efficient performance.[30] used pan and Tompkin's algorithm to identify the QRS complex. This algorithm aims to classify PVC arrhythmia from ECG signals automatically. The proposed algorithm selects the maximum threshold for the cross-correlation coefficients between the generated sub-signals and the template T(n) signals. Although it is efficient, extracting the other relevant features for the PVC arrhythmia classification is required.

Despite the notable efforts done in previous studies to analyze life-threatening arrhythmic PVC beats, existing research solutions utilize a complicated way to analyze the morphological or temporal features. Several existing studies fail to extract many features for PVC arrhythmia identification which analyzes inter-patient variability in ECG signals. In addition, the traditional adaptive threshold method adopted by the Pan and Tompkins algorithms has the problem of incorrect threshold due to the occurrence of interference. It requires multiple prior data points to predict PVC arrhythmias accurately. Most of the studies did not focus on R-peak-related features such as RR interval and previous RR interval standard deviation of the RR interval. However, these are the most crucial features not discussed in previous studies. However, using the conventional technique of the Pan and Tompkins algorithm, those features of PVC arrhythmia cannot be accurately extracted, which has multiform patterns due to the massive variation in ECG signals. Pan and Tompkins's algorithm is only limited to extracting the regular bizarre QRS with positive peaks. However, QRS often has negative peaks that are not extracted using the conventional pan and Tompkin’s algorithm. The conventional Pan and Tompkins algorithm has high complexity [32]. Failing to detect the exact features of ECG signals returns misclassification, which is highly dangerous when the system implements it in real-world scenarios. This study proposed the FAPAC model for feature extraction and classification to bridge this gap, which supports inter-patient variation in ECG signals. This study performed extensive experiments to identify PVC arrhythmia. We assume that the research will point to a new research path in machine learning and the internet of medical things-based applications.

The remainder of the paper is structured as follows: The author explained related work in Section 2. Section 3 discusses the ECG monitoring module and FAPAC model respectively. Sections 4 and 5 discuss the time complexity and result and discussion, respectively. The comparison of the current study with state-of-the-art methods discusses in Section 6. Finally, Section 7 provides the conclusions and future directions.

ECG MONITORING MODULE

Three main components or modules are used to collect the ECG signals in this study to analyze the critical and non-critical heartbeat, including data acquisition, data transmission, and data processing. Each stage used different sensor modules and performed another function represented in Figures 1, 2, and 3. In the initial stage, data collection takes place using the patient skin surface. ECG monitoring modules' supervision in this stage is too much necessary due to the accuracy issue in collecting ECG signals. The efficiency in data collection will determine the highest accuracy in the classification outcomes. Secondly, the data transmission needs to be checked via a microcontroller, and sometimes information is lost due to network communication disturbance. The last stage is processing data on the fog user system according to their task priority. The detailed description of each module is described in the following subsections. Figure 4 presents the schematic design of the ECG monitoring module. The purpose of this monitoring module is to collect data, and it also can transmit data to the cloud for remote monitoring. The detailed workflow of the FAPAC model is described in Figure 5.

2.1 Data Acquisition using AD8232 Module

AD8232 is the main sensor module that collects ECG signals from the patient body surface.AD8232 is an analog sensor with a 3.3V operating power factory-made by Sparkfun. This sensor module contains three terminals of electrodes (Right Arm (RA), Left Arm (LA), and Right Leg RL) which are used to place on the patient body surface for the collection of ECG signals. Furthermore, two built-in analog filters are available in this sensor module, a high pass filter and a low pass filter, which help to minimize the ECG signal noise with cut-off frequencies 0.48Hz and 40Hz, respectively [33]. There are five types of pins present in the AD8232 module, which are 3.3V, GND, Output (returns ECG signal), LO- and LO+ (sends information related to the electrode information loss and connections), and SDN (works as a battery saver and for shutdown). It is mandatory to put the patient to rest condition for the perfect collection of ECG signals. Einthoven’s Triangle placement of AD8232 is illustrated in Figure 1. This module's output is displayed using the Arduino IDE serial monitor (COM) and later is used for the transmitting purpose.

Figure 1: The Einthoven’s Triangle placement of the AD8232 Module on the body surface[1].

2.2 Data Transmission using the ESP8266 WLAN Module

The proposed ECG IoMT-based ECG monitoring system is developed using ESP8266 produced by Espressif Systems in Shanghai, China. It is a low-cost module chip with a 32-bit RISC processor that is widely used in the rapid development of intelligent sensor systems and IoT-based solutions. The programming of this module is based on AT commands. The purpose of using this WiFi module is to connect with the microcontroller directly to transmit ECG signal data on the Fog user system. ESP8266 WiFi module transmits the raw ECG data towards Fog using the wireless channel to the access point. This WiFi module performs as a crucial communication bridge between the ECG signals and data over the network. It has 80% processing capabilities for network-related user systems. The ESP WiFi module is shown in Figure 2.

Figure 2: ESP8266 WiFi module[2]

2.3 Data Processing using Arduino Nano 3 Component

This component plays a vital role in integrating and connecting two different modules to collect, transmit, and process ECG signals. The signal ECG stream series is the reading of the voltage managed by the third component, Arduino nano 3. The central processing unit utilized in IoMT-based ECG monitoring for PVC arrhythmia (critical heartbeat) identification proposed a healthcare system that consists of an ATmega328 microchip as the microcontroller. The graphical representation of the Arduino nano 3 component is shown in Figure 3. Arduino Nano involves a 10-bit built-in analog-to-digital (ADC) converter, reducing system complexity and vital in acquiring and processing analog data signals from the AD8232 chip. Arduino Nano is also equipped with a set of communication terminals for serial data transmission. The output data is transmitted from the AD8232 module to Arduino by connecting the OUTPUT pin to the A1 analog pin. As explained, the data will be processed using an A/D converter and USART to transmit the data serially via pins 2 and 3 to the ESP8266 wi-fi module.

Figure 3: Arduino Nano 3 component[3]

Figure 4: The schematic diagram of the Internet of Medical Things-based ECG monitoring module.

FAPAC Model

This study aims to design a novel solution for the detection of Premature ventricular contraction with all features of the ECG signals which may cause a serious threat to life. In general,[] usually considers the MIT-BIH arrhythmia public benchmark dataset for the identification of different types of arrhythmia, however, those recordings are already preprocessed and do not require heavy preprocessing. One of the drawbacks of using those recordings is that each ECG device has its miscellaneous noises and developing the algorithm using those recordings may fail to integrate with those devices which collect different types of noises as well as information of interest. Therefore, in this study, we have followed a detailed procedure for the efficient preprocessing of completely new recordings which are acquired by our own ECG Monitoring Module. In the first stage of the model we acquire the dataset we also consider the public benchmark dataset for the result comparison, secondly, we have modified the Pan and Tompkins algorithm for the acceptance of negative polarity changes in ECG signals, each feature extraction phase is elaborated in modified Pan and Tompkins section with the detailed workflows. Furthermore, we have tuned the new classifier Cyclic Echo state Networks which has good capability to accept real-time changes. In the end, we evaluate the performance of the proposed FAPAC model.

Figure 5: The detailed workflow of the FAPAC model

3.1 ECG Data Acquisition

Our proposed ECG monitoring module was used for real-time ECG acquisition. A total of thirty ECG subjects with a 20-minute duration inclusive dataset was collected from the Cardiology Ward of Liaquat University Hospital, Jamshoro, Sindh, Pakistan. Furthermore, most of the studies in the literature used the MIT-BIH arrhythmia dataset [37] for the experimental setup. To compare studies fairly with our proposed methodology, we also consider the Public benchmark dataset. The standard division is used to divide the dataset for training and testing purposes which were presented by [38][39]. Table 1 briefly describes the classification of the MIT-BIH arrhythmia dataset. DS1 was used for training and DS2 for testing purposes. MIT-BIH consists of a total of five classes normal beat (N), supraventricular ectopic beats(S), Premature ventricular contraction beat(V), a fusion of ventricular and normal beats(F), and unknown beats(Q).

Table  1:  Class distribution scheme of MIT-BIH-Arrhythmia dataset[4]

| Dataset | Recordings | N | S | V | F | Q | Total Beats |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dataset | 44 | 90127 | 2745 | 7008 | 802 | 17 | 100699 |
| DS1 | 101, 106, 108, 109, 112, 114, 115, 116, 118, 119, 122, 124, 201, 203, 205, 207, 208, 209, 215, 220, 223, 230 | 45880 | 909 | 3788 | 414 | 10 | 51001 |
| DS2 | 100, 103, 105, 111, 113, 117, 121, 123, 200, 202, 210, 212, 213,214, 219, 221, 222, 228, 231, 232, 233, 234 | 44247 | 1836 | 3220 | 388 | 7 | 49698 |

3.2 Modified Pan and Tompkins Algorithm

Pan and Tompkin's algorithm [32] is the conventional technique that has been previously seen for R-peak detection [7][29][41][42]. This study conducts a unique experiment to analyze premature ventricular contraction arrhythmia patterns and detect the different structures of PVC arrhythmia from ECG signals. PVC arrhythmia often occurs between the normal beat (see Figure 6). The conventional Pan and Tompkins algorithm structure is more complex and thus has more implementation steps. Moreover, the amplitude parameter of the ECG signal is an important part of the analysis in the Pan and Tompkins algorithm, and the statistical average value of the ECG signal should be removed. In this study we have modified the algorithm according to the clinical specialist's advice, for instance, to detect premature ventricular contraction heartbeat and inter-patient variability from the ECG signals accurately, the border of the QRS complex area needs to be considered carefully. A differentiation step of Pan and Tompkins helps to identify the gradient information of the QRS area. However, conventional pan and Tompkins did not return the negative polarity values. Therefore, this research aims to experiment with which modifies the pan and Tompkin’s algorithm to determine the accurate R-peak location and extract other relevant attributes for the efficient detection of PVC arrhythmia. The proposed modified Pan and Tompkins algorithm is depicted in Figure 7. The following steps of the algorithm are described as follows:

Figure 6: Premature ventricular contraction beats in the ECG signal[5]

Figure 7: Block diagram of modified Pan and Tompkins algorithm

3.2.1 Signal Filtering

The recorded ECG signals are often contaminated by different kinds of noises, for instance, baseline wandering, power line interference, and instability of electrode-skin contact with the body [44]. The presence of noises and artifacts makes it difficult to analyze the ECG signal properly, making it less accurate in rhythm analysis and classification. The corrupt ECG signal can result in inaccurate evaluation, and it also has a significant impact on the output of algorithms during classiﬁcation [45][46]. The filtering technique is quite challenging due to the spectral overlap between noise and information of interest in the ECG signal. This research implements Finite impulse response as a low pass and median filter to remove the ECG signals' powerline interference and baseline wandering. The median filter is known as the nonlinear digital filter, and it is efficient to implement on one-dimensional time series data. FIR filter is designed to analyze the coefficient from the ECG signals to remove powerline interference. The reason for implementing these two filters together is because the ECG signal often has baseline drift and powerline interference, each filter has different roles to eliminate the noise median used in this study to eliminate the baseline wander outliers, and the FIR filter implemented to eradicate the powerline interference from ECG signal.

3.2.2 Differentiator

After successfully implementing the filtering methods, the quality vector of the ECG subjects was used to derivate to obtain the high slope values. A total of five points of the derivative with a transfer function and gain of 0.1 are estimated. T represents the sampling period and the output of the differentiator. The equation of differentiator equation is defined below:

Hx=18T(-x-2-2x-1+2x1+z2)                                                        (1)

ynT=18T[-xnT-2T-2xnT-T+2xnT+T+x(nT+2T)]                 (2)

3.2.3 Squaring Function

The purpose of this step is to make nonlinear amplification and to make all data points positive value. The output of this step is to get high-frequency components from the ECG signals by using the equation

An      YnT=[x(nT)]2                                                                                     (3)       {\displaystyle H(z)=0.1(-2z^{-2}-z^{-1}+z^{1}+2z^{2})}

3.2.4 Moving Average For QRS Interest Block Detection

Two event-related moving averages are used to create blocks of interest. QRS features and heartbeats are extracted using the first moving average QRSmi and second-moving average HBmi respectively. The created blocks are then subjected to an event-related threshold to separate the blocks that contain R peaks from the ones that include noise. The QRS moving QRSmi average is used to smooth out several peaks corresponding to QRS complex intervals so that the QRS region may be highlighted and extracted:

The              QRSmin=1/QRSad(y[n-(QRSad-1)/2]+…+yn+…+y[n+(QRSad-1)/2])         (4)

QRSad represented as the approximate duration, scaled up to the next odd integer, where n represents the number of data points. Figure 8 describes the interesting area of QRS. Furthermore, the main motive of the one-beat moving average  HBmi is the same as like QRSmi  however, it highlights the QRS beat to be utilized as a limit for the first moving average QRSmi:

HBmin=1/HBad(y[n-(HBad-1)/2]+…+yn+…+y[n+(HBad-1)/2])                (5)

Figure 8: Results of detection of the interesting area of the QRS block

The usage of  HBad without an extra offset, diminishes the detection accuracy due to its sensitivity to a low signal-to-noise ratio (SNR). The SNR is defined as the ratio of a region's mean signal to its standard deviation[6]. This indicates that when the statistical mean of the signal increases, SNR also increases. As a consequence, an offset is introduced using the statistical mean of the signal which is represented as

ε=∂ES                                                                                                                                     (6)

Where ES  represented as the statistical mean of the squared ECG signal where the fraction of the ES , the signal is represented by  ∂, the threshold offset of HBmithe signal is represented by ε . The dynamic threshold calculates to increase the accuracy of detected QRS complexes by using the following equation.

τ1=HBmi n+ ε                                                                                                                           (7)

The comparing of  QRSmi signal with τ1 create the interesting area of QRS. If a created area is higher than                                                                   τ1, it is categorized as a block of interest since it contains ECG main features such as  (P, QRS, or T). At this point, an interesting area of QRS is created and represented by area[n]. The next step is to remove the noise-induced blocks. The rejection should be related to the expected block width.

3.2.5 QRS Point Thresholding

Here, the undesired blocks are rejected by using the new τ2 threshold to reject the blocks that contain P and T waves and noise. By applying the τ2  threshold, the accepted blocks contain only QRS complexes:

τ2=QRSad                                                                                                                                         (8)

The QRS length in arrhythmia ECG signal durations is often varied. Thus, the condition is configured to capture both average (healthy beats) and broad QRS complex lengths (arrhythmia beats). The QRS complex is classified when the block width is higher than or equal to QRSad . If it is not classified as a QRS complex then it considers a P-wave, T-wave, or noise. where N represents the total number of samples in the integration window's width. Hence N is the most important factor in the moving window. In general, the size of the window must be equal to the broadest QRS complex. Sometimes the peaks are too narrow, in that case, QRS and T complexes merge.

| Algorithm 1 Steps of Detection QRS area and R-peak |  |
| --- | --- |
| 1 | - Procedure create function QRS - Input: FiltECG,QRSad,HBad,∂) - Output: Detect QRS area |
| 2 | Squaring_filt=Square (FiltECG) |
| 3 | QRSmi=MI(Squaringfilt,QRSad) |
| 4 | HBmi =MI(Squaringfilt,HBad) |
| 5 | ES=mean(Squaringfilt) |
| 6 | ε=∂ES+HBmi |
| 7 | τ1=HBmi+ ε |
| 8 | for n=1 to length (QRSmi) do |
| 9 | If QRSmin>τ1 Then |
| 10 | Interested area of QRS block=0.1 |
| 20 | Else |
| 21 | Interested area of QRS block=0 |
| 22 | End if |
| 23 | End for |
| 24 | area←onset and offset from Intrested area of QRS |
| 25 | Set τ1=QRSad |
| 26 | For k=1to number of QRSarea do |
| 27 | If width (QRSarea[k])>=τ2 Then |
| 28 | Rpeak←maximum value of the QRSarea |
| 29 | Else |
| 30 | Search inverse peak using algorithm 2 |
| 31 | End if |
| 32 | End for |
| 33 | End Procedure |

3.2.6 R-peak Related Fiducial Markers Extraction

The proposed modified algorithm helps to extract the QRS complex duration and R-peak. These two attributes are the main, by using these features, this study extracts other relevant attributes which are helpful to identify PVC arrhythmia accurately. After the extraction of R-location from the ECG signals, the R–R interval is the most important feature of PVC arrhythmia detection. However, we compute the subsequent R–R interval based on theQRSmi initiate the jth element from 1 to Rpeak and take out the difference between Rpeak(jth +1) and Rpeak(jth). Furthermore, the previous R-R interval is computed for the comparison of the PVC beat and normal beat. The calculation of this parameter is relatively easy. Once we have an R-wave location, we locate the previous R–R interval by using this equation Rpeak (jth-1). Furthermore, due to the frequent occurrence and different structure of PVC arrhythmic beats in the ECG signal, sometimes the value of inverse R peaks is missed to detect therefore to eliminate this issue, this study rechecks the ECG signal for clarification of Rpeak. Figure 9 (a,b) represents the results of detecting the R-peaks based on the modified Pan Tompkins algorithm and the proposed algorithm for computing inverse peaks. The representation of the RR interval is described in Figure 10.

(a)					(b)

Figure 9 (a, b): Represents the regular and inverse R-peak detection

Figure 10: The representation of RR interval and QRS interested block area

3.3 Cyclic Echo State Networks

Echo state networks (ESNs) are a form of RNN that uses a fixed RNN as a pool, has an input layer, and is sampled by the output layer. It is a new type of recurrent neural network which solves the previous issues of the traditional recurrent neural network, and it has a very low cost for the training phase [7]. Figure 11 describes the architecture of cyclic echo state networks. The basic idea is to convert the low-dimensional time input into a high-dimensional state, and then train the weights of the output connections, so that the system outputs the target information. Training is generally faster and more computationally efficient than other recurrent neural networks because only the output weights change. Reservoir Computing based architecture consists of three primary layers. The input layer helps feed the input signal to the classifier, reservoir states. The last layer is output, which allows combining the reservoir states linearly and the output of the reservoir states. The non-linear way of inputting the feature vector helps the reservoir to efficiently compute its task with a wide range of time-series data. In general, the ESN reservoir takes O(Nx2)  time complexity for each time step and for all data it considers O(Nx2 T). Whereas Nx  consider as input feature vector set and T represents the length of time series data.

Figure 11: Architecture of proposed cyclic or ring ESN

The detection of normal and abnormal attributes from the entire heartbeat is formalized as supervised learning which demands the binary output ECG signals, however, the time duration of classifying the ventricular heartbeat detection task is defined by   Ytargeti(T).

riT=βYtargetiT+1-βycesn.T                                                                                      (9)

Ytargeti(T)=1 T∈VEB   0 T∈SVEB                                                                                                    (10)

The target matrix for calculating the Wout in each cycle i, the vector riT is used. The estimation of the target is generated by Ytarget(T). This new strategy of training improves the learning of the classifier. However, extra training cycles are required when the duration of training is increasing.  To train the network, the feedback controller is utilized and the feedback gain was decreased to 0 for 10 epochs. Table 2 defines the cyclic echo state network hyperparameter setting. Table 3 summarises the experiment details including the value of the number of epochs and 𝛽 value, where the cesn was trained for 10 epochs. Compared to linear activation functions, it performs better than nonlinearity.

Table 2: Hyperparameters of the cyclic echo state networks.

| CESN Hyper Parameter | Setting |
| --- | --- |
| Input Dimension Fn | 13 |
| Reservoir dimensionNx | 300 |
| Readout layer dimensionNout | 13 |
| Input radiusσin | 0.7 |
| Input DensityNd | 0.7 |
| Reservoir density(Wden) | 0.6 |
| Reservoir radius(σfm) | 0.6 |
| Feedback densitydenfb | 0.7 |
| Feedback radius(σfb) | 0.6 |
| Reservoir spectral radius (ρ) | 0.6 |
| Leaking rate α | 0.1 |
| Normalization constant(𝛽) | 0.000001 |
| Bias(Wbias) | 0.03 |

Table 3: 10-fold cross-validation experiment details of cyclic echo state networks

| Experiment no. | 𝛽 | cpe |
| --- | --- | --- |
| K=1 | 1 | 1 |
| K=2 | 0.1 | 1 |
| K=3 | 1 | 1 |
| K=4 | 0.3 | 2 |
| K=5 | 0.04 | 3 |
| K=6 | 0.1 | 5 |
| K=7 | 0.02 | 1 |
| K=8 | 0.1 | 2 |
| K=9 | 0.3 | 10 |
| K=10 | 0.05 | 6 |

**cpe: cycle per epoch

4. Time Complexity

This study aims to detect the ventricular heartbeat for the classification of PVC arrhythmia. It is quite challenging to calculate the classifier's actual time with the unbalanced dataset for the detection of PVC arrhythmia. Since in this case, window-based segmentation has taken place for the pattern recognition of the heartbeat, the main reason for doing this window-based segmentation is to accurately identify ECG signals' patterns for minimizing the inter-patient variability issue. The Output Y(T) vector has the total dimension of output classes, whereas YtargetiT=1 for the entire segmented heartbeat which is related to the PVC class and YtargetiT=0 for the rest of the other classes present in the ECG signal. The model is trained according to Ytargeti and the time duration of classifying each subject is defined by

Ypredicted(T)=i   argmax     ∆Tτ+∆TQ=T+τ2∆TT+τ2∆TyiQ∆T                                                                         (11)

iargmax   yi                                                                                                                                          (12)

The ith  dimension of the yi(T) is represented as y(T)  which is obtained from the CESN model u(T) and τ defined as an integration interval that is adjusted according to the 10th fold cross-validation, whereas yi(T) describes the average time of classifying the heartbeats over the window-based segmented heartbeat τ  centered at T is computed as:

y=∆Tτ+∆TQ=T+τ2∆TT+τ2∆TyiQ∆T                                                                                             (13)

=∆Tτ+∆TQ=T+τ2∆TT+τ2∆TWoutuQ∆T;xQ∆T                                                                                (14)

=Wout∆Tτ+∆TQ=T+τ2∆TT+τ2∆TuQ∆T;xQ∆T                                                                                           (15)

=WoutA                                                                                                                                   (16)

The Ridge regression technique is used to reduce the overfitting of the model; while testing, this study only involves sparse connectivity. The reason for applying sparse connectivity is to minimize the heavy calculation of the model. The total time monitored while implementing the whole FAPAC model is around 1.5s in extraction features, and 5sec is only used to identify the heartbeat. However, the total computational time monitored for training is approximately 8 minutes.

5. Result and Discussion

A ten-fold cross-validation technique is used to validate the FAPAC model and the standard performance measures were used to analyze the performance of the purposed cyclic echo state networks. To evaluate the performance of our FAPAC model, the study used three performance parameters which are recommended by AAMI standards for evaluating the performance of the learning algorithm, which include classification accuracy (ACC), sensitivity (Se), specificity (Sp), positive predictivity(+PPV), F1 score, the area under the curve (AUC), G-mean and error rate (Er). The equations are as follows:

Acc=1Ni=1N(TP+TN)i(TP+FP+TN+FN)i*100                                                       (17)

Se=1Ni=1N(TP)i(TP+FN)i*100                                                              (18)

DEr=1Ni=1N(FP+FN)i(TP+TN+FN+FP)i*100                                                       (19)

Where True negative (TN) = correctly classified normal record, True positive (TP)= correctly classified abnormal record, False positive (FP)= normal records classified as abnormal record, and False Negative (FN) = abnormal records classified as a normal record.

5.1 Inclusive Dataset Results

This experiment aims to reveal performance for real-time analysis. A total of thirty ECG subjects with a 20-minute duration inclusive dataset were collected from the Cardiology Ward of Liaquat University Hospital, Jamshoro Sindh, Pakistan. Before the collection of the inclusive dataset, we have taken ethical consent from all patients. The initial criteria for analysis of critical and non-critical heartbeats were set based on the R-R interval and QRS complex using these parameters; other relevant features are extracted to distinguish the critical and non-critical subjects. The detailed description and extracted values of the prominent peak for identifying critical and non-critical subjects of the collected ECG dataset are defined in Table 4. To evaluate the classification performance of our proposed FAPAC model, the standard parameters were used.  Table 5 and Table 6 discuss the detailed results of the FAPAC model using inclusive and public datasets. However, the real names of the patient and real information about heart disease are confidential. Therefore, the table only shows the information about PVC arrhythmia beat. However, the ranges of critical and non-critical are already discussed above. Among the ten ECG subjects, three ECG recordings were noted as critical due to the frequent occurrence of PVC arrhythmia. According to our classifier, the recorded ECG subject CHL004 seems less critical than CHL007 and CHL0010. Whereas in two recordings CHL007 and CHL0010 patients have severe heart diseases, the ratio of PVC heartbeats is more in less time duration recording only. Table 5 shows the real-time collected ECG dataset's classification performance using IoMT based ECG monitoring module. Furthermore, it is also noticed that affected ECG subjects also have inter-patient variability issues, which are successfully classified using our proposed classification technique and achieved positive predictivity and sensitivity around 98.89% and 98.87%, respectively. Whereas, the error rate is identified using the error rate formula, which is around 0.0018%.

Table 4: Description and Feature Extraction results of the real-time collected ECG dataset.

| ECG Subject | Gender | age | R-R Interval | QRS Interval | Condition Analyze |
| --- | --- | --- | --- | --- | --- |
| CHL001 | Male | 43 | 0.874 | 0.092 | Non-critical |
| CHL002 | Male | 25 | 0.746 | 0.086 | Non-critical |
| CHL003 | Male | 32 | 0.987 | 0.089 | Non-critical |
| CHL004 | Male | 65 | 1.057 | 0.233 | Critical |
| CHL005 | Male | 52 | 0.103 | 0.096 | Non-critical |
| CHL006 | Female | 60 | 0.875 | 0.087 | Non-critical |
| CHL007 | Male | 48 | 0.518 | 0.165 | Critical |
| CHL008 | Male | 23 | 0.749 | 0.085 | Non-critical |
| CHL009 | Male | 55 | 0.932 | 0.099 | Non-critical |
| CHL0010 | Female | 50 | 0.47 | 0.172 | Critical |
| CHL0011 | Male | 35 | 1.03 | 0.354 | Critical |
| CHL0012 | Male | 42 | 1.3 | 0.146 | Critical |
| CHL0013 | Female | 40 | 1.5 | 0.162 | Critical |
| CHL0014 | Male | 47 | 0.95 | 0.080 | Non-critical |
| CHL0015 | Male | 50 | 1.01 | 0.154 | Critical |
| CHL0016 | Male | 39 | 0.98 | 0.176 | Critical |
| CHL0017 | Female | 40 | 0.90 | 0.090 | Non-critical |
| CHL0018 | Female | 52 | 0.97 | 0.192 | Critical |
| CHL0019 | Male | 57 | 1.02 | 0.183 | Critical |
| CHL0020 | Female | 37 | 0.75 | 0.082 | Non-critical |
| CHL0021 | Male | 39 | 0.69 | 0.085 | Non-critical |
| CHL0022 | Female | 40 | 0.80 | 0.087 | Non-critical |
| CHL0023 | Male | 55 | 1.4 | 0.133 | Critical |
| CHL0024 | Female | 63 | 0.25 | 0.153 | Critical |
| CHL0025 | Male | 70 | 0.59 | 0.124 | Critical |
| CHL0026 | Female | 41 | 0.86 | 0.090 | Non-critical |
| CHL0027 | Female | 45 | 0.70 | 0.092 | Non-critical |
| CHL0028 | Female | 65 | 0.16 | 0.139 | Critical |
| CHL0029 | Female | 62 | 0.13 | 0.163 | Critical |
| CHL0030 | Male | 69 | 0.17 | 0.191 | Critical |

Table 5:The classification performance of real-time collected ECG dataset

| Rec no | Total Beats | PVC Beats | FP | FN | +P | Se (%) | Der(%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHL001 | 1547 | 0 | 1 | 2 | 99.93 | 99.87 | 0.001 |
| CHL002 | 1493 | 0 | 2 | 2 | 99.86 | 99.86 | 0.002 |
| CHL003 | 1494 | 0 | 1 | 1 | 99.93 | 99.93 | 0.001 |
| CHL004 | 1524 | 105 | 1 | 2 | 99.93 | 99.86 | 0.001 |
| CHL005 | 1592 | 0 | 2 | 3 | 99.87 | 99.81 | 0.003 |
| CHL006 | 1511 | 0 | 1 | 3 | 99.93 | 99.80 | 0.002 |
| CHL007 | 1561 | 200 | 5 | 10 | 99.68 | 99.36 | 0.009 |
| CHL008 | 1459 | 0 | 3 | 5 | 100 | 99.65 | 0.005 |
| CHL009 | 1505 | 0 | 0 | 3 | 100 | 99.80 | 0.001 |
| CHL0010 | 1487 | 130 | 3 | 5 | 99.79 | 99.66 | 0.005 |
| CHL0011 | 1470 | 20 | 1 | 1 | 99.93 | 99.93 | 0.001 |
| CHL0012 | 1495 | 200 | 4 | 2 | 99.73 | 99.86 | 0.004 |
| CHL0013 | 1444 | 5 | 0 | 0 | 99.65 | 100 | 0 |
| CHL0014 | 1416 | 0 | 1 | 0 | 100 | 100 | 0.0007 |
| CHL0015 | 1442 | 60 | 0 | 3 | 100 | 99.79 | 0.002 |
| CHL0016 | 1510 | 35 | 2 | 0 | 99.86 | 100 | 0.001 |
| CHL0017 | 1500 | 0 | 0 | 0 | 100 | 100 | 0 |
| CHL0018 | 1519 | 28 | 1 | 2 | 99.93 | 99.86 | 0.001 |
| CHL0019 | 1518 | 91 | 2 | 0 | 99.86 | 100 | 0.001 |
| CHL0020 | 1542 | 0 | 3 | 0 | 99.80 | 100 | 0.001 |
| CHL0021 | 1550 | 0 | 0 | 0 | 100 | 100 | 0 |
| CHL0022 | 1589 | 0 | 0 | 0 | 100 | 100 | 0 |
| CHL0023 | 1654 | 108 | 0 | 1 | 100 | 99.93 | 0.0006 |
| CHL0024 | 1687 | 20 | 2 | 0 | 99.88 | 100 | 0.0011 |
| CHL0025 | 1699 | 75 | 3 | 1 | 99.82 | 99.94 | 0.002 |
| CHL0026 | 1634 | 0 | 1 | 2 | 99.93 | 99.87 | 0.001 |
| CHL0027 | 1564 | 0 | 0 | 0 | 100 | 100 | 0 |
| CHL0028 | 1582 | 150 | 2 | 1 | 99.87 | 99.93 | 0.001 |
| CHL0029 | 1587 | 300 | 4 | 1 | 99.74 | 99.93 | 0.003 |
| CHL0030 | 1578 | 550 | 2 | 5 | 99.87 | 99.68 | 0.004 |
| Total/Avg | 46153 | 2077 | 47 | 55 | 99.89 | 99.87 | 0.0018 |

6. Comparison with State-of-the-art Methods

The performance of the proposed FAPAC model using the public benchmark dataset is described in Table 6. To promote fair comparison we have chosen a public benchmark dataset. In total 100694 beats were identified of which 93746 were normal, 6933 were successfully identified as PVC and 15 beats were missing beats. Moreover, our proposed study successfully achieved the highest performance in terms of sensitivity, and accuracy which are around 99.98% and 99.97% respectively. The detection error rate was observed at 0.025 which is very less. Thus, according to the result analysis, the FAPAC model is also capable to support other different types of recordings for the identification of abnormalities from ECG signals which support the same features of ECG signals. Table 7 highlights the comparison of the proposed model with an existing technique to classify PVC arrhythmia from ECG signals using supervised learning and unsupervised learning technique.

Table 6: Results of ventricular heartbeat classification using MIT-BIH-Arrhythmia full dataset.

| Records | Beats | N | PVC | MB | Se% | ACC% | DER% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 2272 | 2271 | 1 | 0 | 100 | 100 | 0 |
| 101 | 1862 | 1862 | 0 | 0 | 100 | 100 | 0 |
| 103 | 2084 | 2084 | 0 | 0 | 100 | 100 | 0 |
| 105 | 2570 | 40 | 2530 | 0 | 100 | 100 | 0 |
| 106 | 2026 | 1505 | 519 | 2 | 99.90 | 99.80 | 0.19 |
| 108 | 1762 | 17 | 1745 | 0 | 100 | 100 | 0 |
| 109 | 2532 | 2495 | 37 | 0 | 100 | 100 | 0 |
| 111 | 2122 | 2121 | 1 | 0 | 100 | 100 | 0 |
| 112 | 2538 | 2538 | 0 | 0 | 100 | 100 | 0 |
| 113 | 1794 | 1794 | 0 | 0 | 100 | 100 | 0 |
| 114 | 1878 | 1835 | 43 | 0 | 100 | 100 | 0 |
| 115 | 1952 | 1952 | 0 | 0 | 100 | 100 | 0 |
| 116 | 2412 | 2302 | 107 | 3 | 99.91 | 99.83 | 0.16 |
| 117 | 1534 | 1534 | 0 | 0 | 100 | 100 | 0 |
| 118 | 2276 | 2260 | 16 | 0 | 100 | 100 | 0 |
| 119 | 1986 | 1541 | 443 | 2 | 99.89 | 99.79 | 0.20 |
| 121 | 1862 | 1861 | 1 | 0 | 100 | 100 | 0 |
| 122 | 2476 | 2476 | 0 | 0 | 100 | 100 | 0 |
| 123 | 1516 | 1514 | 2 | 0 | 100 | 100 | 0 |
| 124 | 1618 | 1571 | 47 | 0 | 100 | 100 | 0 |
| 200 | 2600 | 1772 | 826 | 2 | 99.92 | 99.84 | 0.15 |
| 201 | 1962 | 1764 | 198 | 0 | 100 | 100 | 0 |
| 202 | 2136 | 2118 | 18 | 0 | 100 | 100 | 0 |
| 203 | 2978 | 2534 | 444 | 0 | 100 | 100 | 0 |
| 205 | 2656 | 2586 | 70 | 0 | 100 | 100 | 0 |
| 207 | 1860 | 1649 | 209 | 2 | 99.89 | 99.89 | 0.10 |
| 208 | 2954 | 1963 | 991 | 0 | 100 | 100 | 0 |
| 209 | 3004 | 3003 | 1 | 0 | 100 | 100 | 0 |
| 210 | 2650 | 2455 | 195 | 0 | 100 | 100 | 0 |
| 212 | 2748 | 2748 | 0 | 0 | 100 | 100 | 0 |
| 213 | 3250 | 3030 | 219 | 1 | 99.93 | 99.93 | 0.06 |
| 214 | 2262 | 2006 | 256 | 0 | 100 | 100 | 0 |
| 215 | 3362 | 3198 | 164 | 0 | 100 | 100 | 0 |
| 219 | 2154 | 2091 | 63 | 0 | 100 | 100 | 0 |
| 220 | 2046 | 2046 | 0 | 0 | 100 | 100 | 0 |
| 221 | 2426 | 2030 | 396 | 0 | 100 | 100 | 0 |
| 222 | 2482 | 2482 | 0 | 0 | 100 | 100 | 0 |
| 223 | 2604 | 2132 | 472 | 0 | 100 | 100 | 0 |
| 228 | 2052 | 1692 | 360 | 0 | 100 | 100 | 0 |
| 230 | 2256 | 2255 | 1 | 0 | 100 | 100 | 0 |
| 231 | 1570 | 1568 | 2 | 0 | 100 | 100 | 0 |
| 232 | 1780 | 1779 | 0 | 1 | 100 | 99.88 | 0.11 |
| 233 | 3078 | 2248 | 828 | 2 | 99.93 | 99.87 | 0.13 |
| 234 | 2752 | 2749 | 3 | 0 | 100 | 100 | 0 |
| Avg/Total | 100694 | 93746 | 6933 | 15 | 99.98 | 99.97 | 0.025 |

Table 7: Comparison analysis of the proposed work with the conventional technique

| Studies | Dataset | Method | Acc(%) | Sen(%) | Sp(%) | PPV(%) |
| --- | --- | --- | --- | --- | --- | --- |
| [8] | MIT-BIH-ARR (DS2) | Ensemble ESN | 98.6 | 84.4 | 95.8 | 99.7 |
| [9] | MIT-BIH-ARR (DS2) | Morse Theory | 92.73 | 73.35 | 96.70 | 88.01 |
| [10] | MIT-BIH-ARR (DS2) | RF | 96.38 | 97.88 | 97.56 | 95.46 |
| [11] | MIT-BIH-ARR (DS2) | SVM and KNN | 99.11 | 82.21 | 85.88 | 85.36 |
| Proposed | MIT-BIH-ARR(DS2) | FAPAC | 99.97 | 99.99 | 99.9 | 99.98 |

7. Conclusion

A novel, fully automatic model for PVC arrhythmia classification is developed. This model can assist in pre-processing extracted features and classifying the abnormality in a real-time manner. The proposed model analyzes ECG signals' morphological and temporal features using the modified pan and Tompkins’s algorithm, and Pearson correlation coefficient. The proposed unique feature extraction technique' performance is evaluated using common parameters such as sensitivity, accuracy, and detection error rate. This research strongly encourages private datasets for the experiment because public datasets are already pre-processed and used by many studies for their experiments. This study also compares our FAPAC model using the public benchmark dataset. We observed that our proposed fully automatic model has overwhelmed the performance of other state-of-the-art studies. The future work of our research would be to analyze the different types of arrhythmias and implement this FAPAC model in the Fog network for remote monitoring. Considering federated learning for remote monitoring of patients also plays a vital role in its being implemented for heart disease patients.

Acknowledgment: The authors are thankful to the Deanship of Scientific Research at Najran University for funding this work under the Research Collaboration Funding program grant code (NU/RC/SERC/11/7).

REFERENCES

[1]	A. S. Malek, A. Elnahrawy, H. Anwar, and M. Naeem, “Automated detection of premature ventricular contraction in ECG signals using enhanced template matching algorithm,” Biomed. Phys. Eng. Express, vol. 6, no. 1, p. 015024, 2020, doi: 10.1088/2057-1976/ab6995.

[2]	Q. U. A. Mastoi, T. Y. Wah, R. G. Raj, and U. Iqbal, “Automated Diagnosis of Coronary Artery Disease: A Review and Workflow,” Cardiol. Res. Pract., vol. 2018, 2018, doi: 10.1155/2018/2016282.

[3]	H. Huang, J. Liu, Q. Zhu, R. Wang, and G. Hu, “A new hierarchical method for inter-patient heartbeat classification using random projections and RR intervals,” Biomed. Eng. Online, 2014, doi: 10.1186/1475-925X-13-90.

[4]	Z. Zhang, J. Dong, X. Luo, K. S. Choi, and X. Wu, “Heartbeat classification using disease-specific feature selection,” Comput. Biol. Med., vol. 46, pp. 79--89, 2014, doi: 10.1016/j.compbiomed.2013.11.019.

[5]	G. Garcia, G. Moreira, D. Menotti, and E. Luz, “Inter-Patient ECG Heartbeat Classification with Temporal VCG Optimized by PSO,” Sci. Rep., vol. 7, no. 1, pp. 1--11, 2017, doi: 10.1038/s41598-017-09837-3.

[6]	Z. Zidelmal, A. Amirou, D. Ould-Abdeslam, and J. Merckle, “ECG beat classification using a cost sensitive classifier,” Comput. Methods Programs Biomed., vol. 111, no. 3, pp. 570–577, 2013, doi: 10.1016/j.cmpb.2013.05.011.

[7]	F. A. Elhaj, N. Salim, A. R. Harris, T. T. Swee, and T. Ahmed, “Arrhythmia recognition and classification using combined linear and nonlinear features of ECG signals,” Comput. Methods Programs Biomed., vol. 127, pp. 52–63, 2016, doi: http://dx.doi.org/10.1016/j.cmpb.2015.12.024.

[8]	C. Ye, B. V. K. Vijaya Kumar, and M. Tavares Coimbra, “An Automatic Subject-Adaptable Heartbeat Classifier Based on Multiview Learning,” IEEE J. Biomed. Heal. Informatics, vol. 20, no. 6, pp. 1485--1492, 2016, doi: 10.1109/JBHI.2015.2468224.

[9]	C. Ye, B. V. K. Vijaya Kumar, and M. T. Coimbra, “Heartbeat classification using morphological and dynamic features of ECG signals,” IEEE Trans. Biomed. Eng., vol. 59, no. 10, pp. 2930--2941, 2012, doi: 10.1109/TBME.2012.2213253.

[10]	F. Alonso-Atienza, E. Morgado, L. Fernandez-Martinez, A. Garcia-Alberola, and J. L. Rojo-Alvarez, “Detection of life-threatening arrhythmias using feature selection and support vector machines,” IEEE Trans. Biomed. Eng., vol. 61, no. 3, pp. 832--840, 2014, doi: 10.1109/TBME.2013.2290800.

[11]	M. G. Tsipouras and D. I. Fotiadis, “Automatic arrhythmia detection based on time and time-frequency analysis of heart rate variability,” Comput. Methods Programs Biomed., 2004, doi: 10.1016/S0169-2607(03)00079-8.

[12]	Q. Mastoi et al., “Machine learning-data mining integrated approach for premature ventricular contraction prediction,” Neural Comput. Appl. 2021 3318, vol. 33, no. 18, pp. 11703–11719, Mar. 2021, doi: 10.1007/S00521-021-05820-2.

[13]	Q. ul ain Mastoi, T. Y. Wah, and R. G. Raj, “Reservoir computing based echo state networks for ventricular heart beat classification,” Appl. Sci., vol. 9, no. 4, p. 702, 2019, doi: 10.3390/app9040702.

[14]	Q. U. A. Mastoi, T. Y. Wah, R. G. Raj, and A. Lakhan, “A novel cost-efficient framework for critical heartbeat task scheduling using the internet of medical things in a fog cloud system,” Sensors (Switzerland), vol. 20, no. 2, p. 441, 2020, doi: 10.3390/s20020441.

[15]	Y. P. Chen et al., “An Injectable 64 nW ECG Mixed-Signal SoC in 65 nm for Arrhythmia Monitoring,” IEEE J. Solid-State Circuits, vol. 50, no. 1, pp. 375--390, 2015, doi: 10.1109/JSSC.2014.2364036.

[16]	O. Castillo, P. Melin, E. Ramírez, and J. Soria, “Hybrid intelligent system for cardiac arrhythmia classification with Fuzzy K-Nearest Neighbors and neural networks combined with a fuzzy system,” Expert Syst. Appl., vol. 39, no. 3, pp. 2947--2955, 2012, doi: 10.1016/j.eswa.2011.08.156.

[17]	I. Saini, D. Singh, and A. Khosla, “QRS detection using K-Nearest Neighbor algorithm (KNN) and evaluation on standard ECG databases,” J. Adv. Res., vol. 4, no. 4, pp. 331–344, Jul. 2013, doi: 10.1016/J.JARE.2012.05.007.

[18]	Z. Dokur and T. Ölmez, “Heartbeat classification by using a convolutional neural network trained with Walsh functions,” Neural Comput. Appl., vol. 32, no. 16, pp. 12515–12534, 2020, doi: 10.1007/s00521-020-04709-w.

[19]	D. Giri et al., “Automated diagnosis of Coronary Artery Disease affected patients using LDA, PCA, ICA and Discrete Wavelet Transform,” Knowledge-Based Syst., vol. 37, pp. 274--282, 2013, doi: 10.1016/j.knosys.2012.08.011.

[20]	R. Ghorbani Afkhami, G. Azarnia, and M. A. Tinati, “Cardiac arrhythmia classification using statistical and mixture modeling features of ECG signals,” Pattern Recognit. Lett., vol. 70, pp. 45--51, 2016, doi: 10.1016/j.patrec.2015.11.018.

[21]	S. Sultan Qurraie and R. Ghorbani Afkhami, “ECG arrhythmia classification using time frequency distribution techniques,” Biomed. Eng. Lett., vol. 7, no. 4, pp. 325--332, 2017, doi: 10.1007/s13534-017-0043-2.

[22]	S. L. Oh, E. Y. K. Ng, R. S. Tan, and U. R. Acharya, “Automated diagnosis of arrhythmia using combination of CNN and LSTM techniques with variable length heart beats,” Comput. Biol. Med., vol. 102, no. June, pp. 278–287, 2018, doi: 10.1016/j.compbiomed.2018.06.002.

[23]	Y. Ji, S. Zhang, and W. Xiao, “Electrocardiogram classification based on faster regions with convolutional neural network,” Sensors (Switzerland), vol. 19, no. 11, p. 2558, 2019, doi: 10.3390/s19112558.

[24]	A. Jovic and N. Bogunovic, “Random forest-based classification of heart rate variability signals by using combinations of linear and nonlinear features,” in IFMBE Proceedings, 2010, pp. 29--32, doi: 10.1007/978-3-642-13039-7_8.

[25]	V. Mahesh, A. Kandaswamy, C. Vimal, and B. Sathish, “Random forest classifier based ECG arrhythmia classification,” Int. J. Healthc. Inf. Syst. Informatics, vol. 5, no. 2, pp. 1--10, 2010, doi: 10.4018/jhisi.2010040101.

[26]	M. M. Casas, R. L. Avitia, F. F. Gonzalez-Navarro, J. A. Cardenas-Haro, and M. A. Reyna, “Bayesian Classification Models for Premature Ventricular Contraction Detection on ECG Traces,” J. Healthc. Eng., vol. 2018, 2018, doi: 10.1155/2018/2694768.

[27]	M. H. Mazidi, M. Eshghi, and M. R. Raoufy, “Detection of premature ventricular contraction (PVC) using linear and nonlinear techniques: an experimental study,” Cluster Comput., vol. 23, no. 2, pp. 759–774, 2020, doi: 10.1007/s10586-019-02953-x.

[28]	T. Xie, R. Li, S. Shen, X. Zhang, B. Zhou, and Z. Wang, “Intelligent Analysis of Premature Ventricular Contraction Based on Features and Random Forest,” J. Healthc. Eng., vol. 2019, 2019, doi: 10.1155/2019/5787582.

[29]	M. A. Z. Fariha, R. Ikeura, S. Hayakawa, and S. Tsutsumi, “Analysis of Pan-Tompkins Algorithm Performance with Noisy ECG Signals,” J. Phys. Conf. Ser., vol. 1532, no. 1, p. 012022, 2020, doi: 10.1088/1742-6596/1532/1/012022.

[30]	A. N. Al-shrouf and S. Arabia, “ECG Beat Classification using a Sliding Window and Correlation of the Three-bit Linear Prediction Error Signal,” Wseas Trans. Signal Process., vol. 16, no. 3, pp. 12–18, 2020, doi: 10.37394/232014.2020.16.2.

[31]	Y. Yue, C. Chen, P. Liu, Y. Xing, and X. Zhou, “Automatic Detection of Short-Term Atrial Fibrillation Segments Based on Frequency Slice Wavelet Transform and Machine Learning Techniques,” Sensors (Basel)., vol. 21, no. 16, Aug. 2021, doi: 10.3390/S21165302.

[32]	J. Pan and W. J. Tompkins, “A Real-Time QRS Detection Algorithm,” IEEE Trans. Biomed. Eng., vol. BME-32, no. 3, pp. 230–236, 1985, doi: 10.1109/TBME.1985.325532.

[33]	F. Buendía-Fuentes et al., “High-Bandpass Filters in Electrocardiography: Source of Error in the Interpretation of the ST Segment,” ISRN Cardiol., vol. 2012, 2012, doi: 10.5402/2012/706217.

[34]	Analog Devices, “AD8232 Single-Lead ECG,” Data Sheet, pp. 1–28, 2013, [Online]. Available: www.analog.com/AD8232.

[35]	B. Benchoff, The Current State of ESP8266 Development. Hackaday, 2014.

[36]	A. Aqeel, “Introduction to Arduino Nano,” Eng. Proj., Jun. 2018, Accessed: Jan. 30, 2022. [Online]. Available: https://www.theengineeringprojects.com/2018/06/introduction-to-arduino-nano.html.

[37]	G. Mark, R and Moody, “MIT-BIH arrhythmia database directory,” Cambridge Massachusetts Inst. Technol., 1988.

[38]	P. De Chazal, M. O’Dwyer, and R. B. Reilly, “Automatic classification of heartbeats using ECG morphology and heartbeat interval features,” IEEE Trans. Biomed. Eng., vol. 51, no. 7, pp. 1196--1206, 2004, doi: 10.1109/TBME.2004.827359.

[39]	M. Llamedo and J. P. Martínez, “Analysis of a semiautomatic algorithm for ECG heartbeat classification,” 2011.

[40]	P. De Chazal and R. B. Reilly, “A patient-adapting heartbeat classifier using ECG morphology and heartbeat interval features,” IEEE Trans. Biomed. Eng., vol. 53, no. 12, pp. 2535--2543, 2006, doi: 10.1109/TBME.2006.883802.

[41]	M. Kumar, R. B. Pachori, and U. Rajendra Acharya, “An efficient automated technique for CAD diagnosis using flexible analytic wavelet transform and entropy features extracted from HRV signals,” Expert Syst. Appl., vol. 63, pp. 165--172, 2016, doi: 10.1016/j.eswa.2016.06.038.

[42]	J. Zhu, L. He, and Z. Gao, “Feature extraction from a novel ECG model for arrhythmia diagnosis,” Biomed. Mater. Eng., vol. 24, pp. 2883--2891, 2014, doi: 10.3233/BME-141107.

[43]	E. P. Gerstenfeld and T. De Marco, “Premature Ventricular Contractions.,” Circulation, vol. 140, no. 8, pp. 624–26, Aug. 2019, doi: 10.1161/CIRCULATIONAHA.119.040015.

[44]	M. Kania, M. Fereniec, and R. Maniewski, “Wavelet denoising for multi-lead high resolution ECG signals,” in 6th International Conference on Measurement, MEASUREMENT 2007 - Proceedings, 2007, pp. 30--33.

[45]	B. Chandrakar, O. P. Yadav, and V. K. Chandra, “a Survey of Noise Removal Techniques for Ecg Signals,” Ijarcc, 2013.

[46]	S. Cuomo, G. De Pietro, R. Farina, A. Galletti, and G. Sannino, “A revised scheme for real time ECG Signal denoising based on recursive filtering,” Biomed. Signal Process. Control, vol. 27, pp. 134–144, 2016, doi: 10.1016/j.bspc.2016.02.007.

[47]	N. M. Arzeno, Z.-D. Deng, and C.-S. Poon, “Analysis of First-Derivative Based QRS Detection Algorithms,” IEEE Trans. Biomed. Eng., vol. 55, no. 2, p. 478, Feb. 2008, doi: 10.1109/TBME.2007.912658.

[48]	H. Jaeger, “The ‘echo state’ approach to analysing and training recurrent neural networks,” GMD Rep., vol. 148, no. 34, p. 13, 2001.

[49]	M. Alfaras, M. C. Soriano, and S. Ortín, “A fast machine learning model for ECG-based heartbeat classification and arrhythmia detection,” Front. Phys., vol. 7, p. 103, 2019, doi: 10.3389/fphy.2019.00109.

[50]	J. Faganeli Pucer and M. Kukar, “A topological approach to delineation and arrhythmic beats detection in unprocessed long-term ECG signals,” Comput. Methods Programs Biomed., vol. 164, pp. 159--168, 2018, doi: 10.1016/j.cmpb.2018.07.010.

[51]	S. Raj and K. C. Ray, “Sparse representation of ECG signals for automated recognition of cardiac arrhythmias,” Expert Syst. Appl., vol. 105, pp. 49--64, 2018, doi: 10.1016/j.eswa.2018.03.038.

———

———