Coastal reservoirs as a source of nitrous oxide: Spatio-temporal patterns and assessment strategy

Ping Yanga,b, Miaohui Lua,b, Kam W. Tanga,b,*, Hong Yangc,d,e,**, Derrick Y. F. Lai Chuan Tonga,b Kwok Pan Chunf, Linhai Zhanga,b, Chen Tangb

aKey Laboratory of Humid Subtropical Eco-geographical Process of Ministry of Education, Fujian Normal University, Fuzhou 350007, P.R. China

bSchool of Geographical Sciences, Fujian Normal University, Fuzhou 350007, P.R. China

cCollege of Environmental Science and Engineering, Fujian Normal University, Fuzhou 350007, P.R. China

dCollaborative Innovation Center of Atmospheric Environment and Equipment Technology, Jiangsu Key Laboratory of Atmospheric Environment Monitoring and Pollution Control (AEMPC), School of Environmental Science and Engineering, Nanjing University of Information Science & Technology, 219 Ningliu Road, Nanjing 210044, China

eDepartment of Geography and Environmental Science, University of Reading, Reading, RG6 6AB, UK

fDepartment of Geography, Hong Kong Baptist University, Hong Kong, China

gKey Laboratory of Watershed Geographic Sciences, Nanjing Institute of Geography and Limnology, Chinese Academy of Sciences, Nanjing, 210008, China

hDepartment of Geography and Resource Management, The Chinese University of Hong Kong, Hong Kong, China

- *Correspondence to: tongch@fjnu.edu.cn (Chuan Tong)

- **Correspondence to: hongyanghy@gmail.com (Hong Yang)

- ***Correspondence to: dyflai@cuhk.edu.hk (Derrick Y.F. Lai)

A B S T R A C T

Coastal reservoirs are widely regarded as a viable solution to the water scarcity problem faced by coastal cities with growing populations. As a result of the accumulation of anthropogenic wastes and the alteration of hydroecological processes, these reservoirs may also become the emission hotspots of nitrous oxide (N2O). Hitherto, accurate global assessment of N2O emission suffers from the scarcity and low spatio-temporal resolution of field data, especially from small coastal reservoirs with high spatial heterogeneity and multiple water sources. In this study, we measured the surface water N2O concentrations and emissions at a high spatial resolution across three seasons in a subtropical coastal reservoir in southeastern China, which was hydrochemically highly heterogeneous because of the combined influence of river runoff, aquacultural discharge, industrial discharge and municipal sewage. Both N2O concentration and emission exhibited strong spatio-temporal variations, which were correlated with nitrogen loading from the river and wastewater discharge. The mean N2O concentration and emission were found to be significantly higher in the summer than in spring and autumn. The results of redundancy analysis showed that NH4+-N explained the greatest variance in N2O emission, which implied that nitrification was the main microbial pathway for N2O production in spite of the potentially increasing importance of denitrification of NO3−-N in the summer. The mean N2O emission across the whole reservoir was 107 μg m−2 h−1, which was more than an order of magnitude higher than that from global lakes and reservoirs. Based on our results of Monte Carlo simulations, a minimum of 15 sampling points per km2 would be needed to produce representative and reliable N2O estimates in such a spatially heterogeneous aquatic system. Overall, coastal reservoirs could play an increasingly important role in future climate change via their N2O emission to the atmosphere as water demand and anthropogenic pressure continue to rise.

Keywords: Nitrous oxide (N2O); Spatial heterogeneity; Spatially resolved measurement; Wastewater discharge; Subtropical reservoir; IPCC

1. Introduction

Nitrous oxide (N2O) is a potent greenhouse gas with a global warming potential nearly 300 times that of carbon dioxide (CO2) on a mass basis over a 100-year time horizon (IPCC, 2013). N2O is also considered to be one of the major substances that can destroy ozone in the stratosphere (Ravishankara et al. 2009; Shaaban et al., 2018). The atmospheric averaged N2O concentration reached 331.1±0.1 ppbv in 2018, which is approximately 23% higher than the pre-industrial (before 1750) levels (World Meteorological Organization, 2019). There has been a steady increase in atmospheric N2O at a rate of 0.7–0.8 ppb yr−1 over the past three decades (Davidson 2009; Saikawa et al. 2014; Xiao, et al., 2019a). Quantifying the potential source strength of various ecosystems is fundamental for predicting future N2O emission and climate change (Yang et al., 2020).

Aquatic ecosystems are considered as important sources of N2O emission, contributing for approximately 25%–30% of global N2O emissions (Zhou et al., 2019). As an important component of the Earth's surface water systems, man-made reservoirs, which include those for hydropower, flood management, water supply, and navigation purposes, are now considered as significant contributors of the atmospheric N2O (Beaulieu et al., 2019; Cheng et al., 2019; Guérin et al., 2008; Wang et al., 2017; Yang & Flower 2012). Recent estimates indicated that the amount of N2O emitted from global reservoirs was approximately 0.03−0.07 Tg N yr−1 (Deemer et al., 2016; Maavara et al., 2019). However, the estimate of N2O emission from reservoirs, especially from the ones with anthropogenic disturbances, remains highly uncertain due to the data limitation. Additionally, N2O emissions from reservoirs in different climatic zones exhibited high spatial-temporal variations across and within the systems (Beaulieu et al., 2015; Cheng et al., 2019; Liang et al., 2019; Liu et al., 2011a, 2015, 2017; Musenze et al., 2014; Shi et al., 2020). This is another source of uncertainty in reservoir N2O budget. Thus, more in situ measurements with high spatiotemporal resolution are needed to reduce the uncertainty and to develop more accurate approaches for upscaling to whole reservoir N2O emissions and further large-scale assessments of reservoir N2O fluxes.

The N2O production in reservoir is known to mainly derive from the nitrification and denitrification of terrestrial nitrogen (N). Human activities in the drainage basin, particularly land use change, sewage discharge and agriculture fertilization, have severely altered the N transport of from the terrestrial ecosystem to the aquatic ecosystem (Davidson, 2009; Hosen et al., 2014; Williams et al., 2016), and influenced the N2O production in reservoirs (Wang et al., 2017). Syvitski et al. (2005) estimated over 100 billion metric tons of sediment and up to 3 billion metric tons of organic matter were trapped in the reservoirs in the last five decades. Hence the allochthonous material load by human activities would play a much more important role for accelerating N2O emission from reservoirs. To date, some efforts have been made on the response of N2O emission from reservoirs to the human activities (e.g., agriculture, urbanization sewage discharges) in the basin, especially in China (e.g., Liu et al., 2011a; Wang et al., 2017; Xiao et al., 2019b). However, the data of N2O emissions from reservoirs under the impact of human activities are disproportionately scarce in comparison to the amount and area of reservoirs. More importantly, the majority of existing studies only focused on inland freshwater reservoirs, but few studies have mentioned N2O emission from coastal reservoirs. As coastal reservoirs often experience a higher salinity and inputs of anthropogenic carbon and nutrients than inland ones, N2O production and emission rates might differ between the reservoirs in these two zones. However, there is currently a lack of empirical data to test this hypothesis and advance our understanding of reservoirs in the global N2O cycle.

In order to fill these knowledge gaps, this study researched high spatial resolution measurements of dissolved N2O concentrations in a subtropical coastal reservoir in southeast China over three seasons of a year to estimate the N2O fluxes across the water-atmosphere interface. The specific objectives of this study are to (1) quantify the magnitude of N2O emission fluxes from the coastal reservoirs, (2) assess the spatial variations in diffusive N2O fluxes both within system and among systems, (3) explore the drivers of the spatial variations of coastal reservoir N2O emissions;  and (4) examine how many measurements sities in space were needed to representatively cover the observed spatial variability in coastal reservoir N2O emissions. The results of the present study will provide the scientific basis for the development of global biogeochemical models and national GHG inventories through characterizing N2O emission from the reservoirs in coastal areas.

2. Materials and Methods

2.1. Study Area

The study was conducted in Wenwusha Reservoir (25°49′36″–25°54′00″ N, 119°35′12″–119°38′11″ E), a subtropical reservoir in the coastal area of Fujian Province, Southeast China (Figure 1). The reservoir was built for irrigation and flood-prevention. It has a surface area of 5.2 km2, a total volume of 3.20×108 m3, drainage area of 275 km2, and mean depth of 9.0 m. The reservoir is influenced by a subtropical monsoon humid climate, with a mean annual temperature of 19.3 °C. The average annual precipitation is 1390 mm, most of which (approximately 75%) occurs in the wet season from May to September.

The Wenwusha Reservoir was created by two dams on the Nangyangdong River estuary. We compartmentalized the reservoir into two reservoir zones according to the two dam constructed time, topographic feature, and trophic status (Figure 1). The northern reservoir zone (NRZ) was constructed in 1957, with a surface area of 1.9 km2 and a total volume of 1.40×108 m3. The NRZ is located in highly urbanized districts and is heavily impacted by human activities, e.g. the sewage discharge from aquacultural, industrial, and municipal activities. The NRZ also received large amounts of nutrient input from Nanyangdong river. The southern reservoir zone (SRZ) was constructed in 2004, with a surface area of 3.3 km2 and a total volume of 1.69×108 m3. Around 70% of its catchment is used for agricultural activities (e.g., aquaculture and farming), but its immediate surroundings are partly forested and wetland (Figure 1). The water salinity in NRZ is much lower than that in SRZ (0.4–1.3‰ versus 0.4–3.7‰), as a result of freshwater dilution caused by the surface runoff.

2.2. Sampling strategies

Considering the reservoir size, topographic feature, and pollution source, a total of 21 transects (10 in the NRZ and 11 in the SRZ) were selected to collect samples to characterize the variation across the whole reservoir. According to the lengths of the transects, there were 3-10 sampling sites in the transects (Figure 1). The total number of sample sites was 103 (56 in the NRZ and 47 in the SRZ). These sites were distributed in 5 different sewage loading sectors, namely the industrial effluent loading sector (Sector-I, n = 4), town sewage loading sector (Sector-T, n = 6), river input sector (Section-R, n = 7), aquaculture sewage loading sector (Section-A, n = 22), and non-wastewater loading sector (Sector-N, n = 64). At each site, surface-water samples were collected at a depth of 20 cm. Three in situ whole reservoir surveys were conducted in mid–November 2018, mid–March and mid–June 2019. Each whole reservoir survey was completed in two consecutive days to reduce the biases caused by day-to-day variation. More details about the sampling transects are presented in supporting information Table S1.

2.3. Measurement of dissolved N2O concentration and diffusive flux

2.3.1. Dissolved N2O concentration

Surface water samples for dissolved N2O concentration analysis were collected using a 100-mL syringe equipped with three-way stopcocks. After sample was collected from reservoir, water was quickly transferred into a 55-mL gas-tight glass serum bottle. Prior to sealing the bottle, 0.2 mL saturated HgCl2 solution was injected into the sample bottle for inhibiting microbial activity. The bottle was immediately sealed with an open-topped screw cap equipped with a halobutyl rubber septum to exclude any air bubble (Borges et al., 2018; Xiao et al., 2019a; Yang et al., 2020). All water samples were stored in an ice box, transported to the laboratory within 6 hr, and analyzed within two days of collection.

Dissolved N2O concentrations in the collected water samples were measured following the headspace equilibration method described by Yu et al. (2013, 2017). In the laboratory, ultrahigh purity N2 gas (99.999%) was injected into the sample bottle via a syringe to create a 25-mL headspace. The bottles were then shaken vigorously for 10 min in an oscillator to allow the dissolved N2O diffuse out and gases reach equilibrium between the liquid phase and headspace. After waiting for 0.5 hr, approximately 5 mL air sample was drawn from the headspace using a syringe equipped with three-way stopcocks. N2O concentrations in the headspace air were measured using gas chromatography (GC-2014, Shimadzu, Kyoto, Japan) equipped with an electron capture detector (ECD). The detection limit for the N2O analysis was 0.02 ppm, and the relative standard deviations of N2O analyses were ≦5.0% (Yang et al., 2020). Based on the equilibrium temperature, salinity–dependent Henry's law constant, and the measured headspace gas concentration, dissolved N2O concentration was calculated using the formula provided by Weiss and Price (1980).

2.3.2. N2O flux from the transfer coefficient method

The N2O flux (FW-A, μmol m−2 h−1) across the water–atmosphere interface was calculated using the classic boundary-layer model (equation (1)), which has been widely used for N2O emissions in the lentic ecosystem (e.g., lakes, reservoir and ponds) (e.g., Cole and Caraco, 1998; Liang et al., 2019; Musenze et al., 2014; Xiao et al., 2019b; Yang et al., 2015a).

FW-A=k×(Cobs−Ceq)                                                 (Eq1)

where Cobs is the measured dissolved N2O concentration (nmol L-1) in the surface water (20 cm depth); Ceq in water that is in equilibrium with the atmosphere at the in situ temperature (nmol L-1); and k is the gas transfer velocity (m h-1).

In the lentic ecosystem, the N2O transfer velocity is mainly driven by wind speed because no surface water flow (Xiao et al., 2019b). k in the present study was calculated using a wind-dependent formula (equation (2)) derived from a small shallow lake as follows (Cole and Caraco, 1998):

k=(Sc660)−n(2.07+0.215×U101.7)                                 (Eq2)

where n = 0.66 and 0.50 for wind speeds ≤3 and >3 m s−1, respectively; Sc is the Schmidt number for N2O and is dependent on the temperature (T, oC); and U10 is the frictionless wind speed at 10 m high expressed in m s−1.

The Sc and U10 were estimated using the following equations (Crusius and Wanninkhof, 2003; Wanninkhof, 1992):

Sc=2055.6−137.11T+4.3173T2−0.054350T3                   (Eq3)

(Eq4)

where Uz is the wind speed (m s-1) at height z (2.0 m in this study) above the water surface at which wind speed was measured; Cd10 is the drag coefficient at 10 m above the water surface (0.0013 m s-1); and K is the von Karman constant (0.41).

2.4. Measurement of environmental variables

During each campaign, from 103 sites in the reservoir was also sampled for measuring chemical properties. Surface water samples were collected at a depth of 20 cm using a 5 L organic glass hydrophore, and transferred into 150 mL polyethylene bottles. Approximately 0.5 mL saturated HgCl2 solution was added to each bottle to inhibit microbial activities (Taipale and Sonninen 2009; Zhang et al., 2013). All samples were preserved in an ice box, and transported to the laboratory for measurement. After filtering the samples through a 0.45 μm cellulose acetate filter (Biotrans™ nylon membranes), the concentration of total dissolved nitrogen (TDN), nitrate-nitrogen (N–NO3–), and ammonium-nitrogen (N–NH4+) was measured using a flow injection analyzer (Skalar Analytical SAN++, The Netherlands). The detection limits for TDN, NN–NO3–, and N–NH4+ were 3.0, 0.6 and 0.6 μg L-1, respectively. The relative standard deviations of TDN, NN–NO3–, and N–NH4+ analyses were ≦2.0%, ≦3.0% and ≦3.0%, respectively.

Water temperature (WT), dissolved oxygen (DO), pH, electrical conductivity (EC), and salinity at a depth of 20 cm were also measured in situ at each sampling site. WT and pH were measured using a portable pH/mV/Temperature meter (IQ150, IQ Scientific Instruments, USA). DO, EC and salinity were determined using a multiparameter water quality probe (550A YSI, USA), an electrical conductivity meter (2265FS EC, Spectrum Technologies, USA) and a salinity meter (Eutech Instruments-Salt6, USA), respectively. The relative standard deviations of DO, pH, EC, and salinity analyses were ≦2.0%, ≦1.0%, ≦1.0% and ≦1.0%, respectively. Meanwhile, the meteorological factors, including the air temperature (AT), atmospheric pressure (AP) and 2 m height wind speed (WS), were recorded using a portable weather meter (Kestrel-3500, USA).

2.5. Statistical analysis

The analysis of variance (ANOVA) to test for significant (p<0.05) effects of reservoir zone and sewage sector water depth on surface water physicochemical properties, dissolved N2O concentration and N2O diffusive fluxes using the SPSS 17.0 statistical software package (SPSS Inc., USA). Pearson correlation coefficients were used to examine the relationships between water physicochemical parameters and N2O concentration/ fluxes. Principal component analysis (PCA) was also performed to analyze the relationships between the N2O fluxes and surface water physicochemical properties to show their pattern at different sampling campaigns. For quantifying spatial heterogeneity effects by randomizing the high-resolution N2O measurements, a Monte Carlo analysis was used to evaluate the effect of sample size on the N2O estimation of the whole reservoir. Without replacements, the N2O measurements are resampled from the 103 sites (n = 10, 20, 30…, 100). The resampling process then was repeated 10000 times and the overall mean and standard deviation of N2O emissions were calculated for each sample size. The results were presented as means ± SE. Statistical plots were generated using OriginPro 7.5 (OriginLab Corp. USA) and ArcGIS 10.2 (ESRI Inc., Redlands, CA, USA). Conceptual diagrams were drawn using EDraw Max version 7.3 (EdrawSoft, Hong Kong, China).

- Results

3.1. Water Quality Parameters in Reservoir

There were substantial spatial variations in water chemical parameters (e.g., salinity, DO, and nutrient concentrations) between both reservoir zones on all three sampling occasions (p<0.01; Table S2), with lower levels of DO (Figure 2a-c), EC (Figure 3g-i and Figure S1d) and salinity (Figure 2j-l and Figure S1c) and higher concentrations of N–NO3− (Figure 2d-f and Figure S2a-c), N–NH4+ (Figure 2g-i and and Figure S2d-f), and TDN (Figure 2j-l and Figure S2g-i) in NRZ than in SRZ. However, the WT (Figure 3a-c) and pH (Figure 3d-f) were relatively constant across the reservoir (p>0.05; Figure S1a and b), with the coefficients of variation only ranged from 4% to 7%, and 1% to 7% over the three sampling campaigns, respectively.

The mean DO, salinity, N–NO3−, N–NH4+ and TDN also differed significantly among the five sewage loading sectors over the three sampling campaigns (Table S3). Mean N–NO3−, N–NH4+ and TDN concentrations were significantly lower in Sector-N than the other sectors (p<0.05;Table S3), and were generally highest in Sector-R. In contrast, mean DO concentration and salinity was generally greater in Sector-N than the other four sectors (Table S3).

The main water quality parameters also showed strong temporal variation across the whole reservoir (p<0.001, Two-way ANOVAs; Table S2). The water temperature was highest in summer (29.3 oC), followed by spring (23.1 oC) and autumn (18.3 oC) (Figure 3a-c). The seasonal patterns of N−NO3− (Figure 2d-f), TDN (Figure 2j-l), pH (Figure 3d-f) and salinity (Figure 3j-l), concentrations were contrary to that of water temperature (Figure 3a-c). The DO varied seasonally, with the peak in spring (Figure 2a-c). In contrast, N−NH4+ concentrations were significantly lower in spring (Figure 2g-i).

3.2. Spatial Variation in Surface Water N2O Concentration

Across all sampling sites in the reservoir, dissolved N2O concentration in November 2018, March 2019 and June 2019 ranged from 8.99 to 86.58, 6.11 to 110.37, and 7.33 to 261.27 nmol L−1 (Figure 4), respectively, with mean N2O concentration of 46.42±2.03, 58.96±5.90, and 43.96±2.69 nmol L−1 in the three sampling campaigns. Across the three sampling campaigns, the mean N2O concentration were significantly greater in NRZ than in SRZ (p<0.05, Two-way ANOVAs; Table 1 and Figure S3a-c). Meanwhile, dissolved N2O concentration also showed remarkably differences betwee the wastewater loading area and the non-wastewater loading area (p<0.05; Table S4 and Figure S4a-c). Across the three sampling campaigns, specifically, the mean N2O concentration were significantly lower in Sector-N than the other sectors (p<0.05; Figure 5a), and were generally highest in Sector-R (Figure 5a).

3.3. Spatial Variation in Diffusive N2O Fluxes Across the Water-Air Interface

Across all sampling sties, the N2O fluxes ranged from 2.61 to 415.61, from -10.83 to 254.01, and from 2.04 to 711.50 μg m−2 h−1 in November 2018, March 2019 and June 2019, respectively, with mean N2O fluxes of 110.41±9.31, 78.48±6.27, and 153.12±8.61 μg m−2 h−1 in the three sampling campaigns. Overall, the Wenwusha Reservoir acted as a constant source of N2O to the atmosphere.

N2O fluxes (Figure 6) showed a similar spatial variability to N2O concentration (Figure 4). Relatively higher N2O fluxes were found in the NRZ of the Wenwusha Reservoir. Across all sampling campaigns, the N2O fluxes in the NRZ ranged from 6.87 to 711.50 μg m−2 h−1, with an average of 153.12±8.61 μg m−2 h−1, which was almost 3 times higher than those from the SRZ (p<0.001, Two-way ANOVAs; Table 1 and Figure S3d-f).

Significant differences in mean N2O emission fluxes were also observed among the wastewater loading area and the non-wastewater loading area (p<0.05; Table S4 and Figures S4d-f). Across the three sampling campaigns, specifically, Sector-N had significantly lower mean N2O fluxes than the other four wastewater loading sectors over the whole study period (Figure 5b), while Sector-R had relatively higher mean N2O fluxes than other sectors (Figure 5b).

3.5. Water Quality Drivers of Variation in N2O Concentrations and Fluxes

The simple regressions analysis demonstrated thatthe spatial variation of N2O concentration and fluxes was positively related to the N−NO3− (r2 = 0.20−0.87, p<0.01), N−NH4+ (r2 = 0.17−0.42, p<0.01 or <0.05) and TDN concentrations (r2 = 0.13−0.88, p<0.01) within each sampling occasion (Table S5, and Figures S5 and S6), respectively, and negatively correlated with that of salinity and DO (p<0.05 or <0.01; Table S5). Principal component analysis (PCA) was also employed to explore the relationships among all these environmental variables over whole reservoir survey during each sampling campaign (Figure 7a-c). The first component (PCA I), explaining 93.1% (Figure 7a), 95.8% (Figure 7b) and 95.7 (Figure 7c) of the total variances during the autumn, spring and summer surveys, respectively, was significantly correlated with N−NO3−, N−NH4+ and TDN. The first component can be considered as the influence of water pollution, reflecting the spatial variation. The second component (PCA II), explaining 3.6%, 3.8% and 4.2% of the variances during the autumn, spring and summer surveys, respectively, was correlated with the EC, salinity and DO. In addition, PCA was employed to explore the temporal correlation between the N2O concentration (or fluxes) and environmental variables. The first and second components together explained 98.2% of the total variance (Figure 7d). In particular, the PCA I contributed 93.8% of the total variance and was correlated with the N−NO3−, N−NH4+, TDN and temperature (Figure 7d).

3.6. Effects of Different Sampling Size on the Estimation of Whole-Reservoir N2O Fluxes

The results of Monte Carlo analysis showed that the accuracies of average diffusive N2O fluxes across the whole reservoir increased considerably with the expansion of numbers of sampling sites included in flux estimations (Figure 8). For each sample size, the resampling process was repeated 10,000 times to create the boxplots of N2O resampling means (Figure 8a) and the standard deviation of resampling means  (Figure 8b).

In Figure 8a, the light grey, grey and dark grey shaded areas show the respective 70%, 95%, and 99% of the Highest Probability Density (HPD) ranges which indicate the variance of standard errors for approximate coverages of the resampling means from 10,000 simulations for each sample size. With the increase of sample sizes, the converging HPD ranges of possible resampling mean N2O values indicated the improving estimation accuracy of N2O flux. In addition, with the expansion of sample sizes, the biases of the resampling N2O estimations declined, and the standard deviations of resampling means decreased exponentially (Figure 8b). Notably, the results of Monte Carlo analysis showed that increase of the sample size from 10 to 100 markedly reduced the relative standard deviation of mean diffusive N2O flux from 21% to 1.2%.

The number of samples in Figure 8 could be used to determine an optimal sample size to produce reasonable flux estimates. However, to achieve less than 5% of error, more than 80 samples over the whole reservoir were still needed (ca. 15 samples per km2), due to reservoir heterogeneity. If the sample sizes were less than 80, the uncertainty range of the resampling simulations increases because of missing some measurements from the N2O hotspots. Therefore, it is important to conduct high spatial resolution sampling to include all N2O hotspots to improve the accuracy of flux estimates.

According to the 10,000 simulations of only 10 samples, a relative standard deviation of the estimated mean diffusive N2O flux was 22.6% (Figure 8b). Despite a large range, these means had a median value of 107 μmol m−2 h−1 (Figure 8a). The bias of the resampling median is small compared to the mean of all samples. Therefore, the high spatial resolution measurements from different transects can effectively reduce the estimation bias of N2O flux in the current study.

Moreover, the standard deviations of resampled N2O mean diffusive flux decreased dramatically from less than 10 μmol m−2 h−1 for the sample size of 40 to 1.29 μmol m−2 h−1 for the sample size of 100 (Figure 8b). The relative standard deviation of mean diffusive flux reduced to only 1.2% for the sample size of 100, indicating that the estimated whole-reservoir N2O emissions approached the actual mean, with much less variation. Increasing the sample size could effectively reduce the estimation bias of the whole-reservoir N2O emissions.

- Discussion

4.1. Subtropical Coastal Reservoir are Atmospheric N2O Sources

Results from our spatially intensive sampling showed that the average N2O emission flux from Wenwusha Reservoir were 107.0 µg m-2 h-1. Although, the average N2O fluxes in the Wenwusha Reservoir were smaller than those in some tropical reservoirs (Table S6), for example, Tucuruiand reservoir, Samuel reservoir (Lima et al., 2002), Petit Saut reservoir and Fortuna reservoir (Guérin et al., 2008), the N2O fluxes in our reservoir were one to two orders of magnitude higher than those in many reservoirs in subtropical (Liu et al., 2011a; Musenze et al., 2014; Shi et al., 2020; Yu et al., 2018) and temperate regions (Cheng et al., 2019; Descloux et al., 2017; Huttunen et al., 2002) (Table S6). The mean N2O emission rates in our reservoir were also substantially higher than the median emission rate (7.2 µg m-2 h-1) in global lakes/reservoirs (Hu et al., 2016), and were approximately 2.5 times higher than the average of China's reservoirs (42.3 µg m-2 h-1) (Li et al., 2018). Compared with lakes worldwide (Table S6), our reservoir had a larger N2O emission fluxes. For example, the mean N2O emission rates in our reservoir was approximately 8.7 times higher than the average of China's lakes (12.2 µg m-2 h-1) (Li et al., 2018). In addition, the mean N2O emission from our reservoir was approximately 3.9 and 1.9 times higher than emissions from Cyperus malaccensis marsh (Wang et al., 2018) and aquaculture ponds (Yang et al., 2020) in the same subtropical coastal region. Futhermore, high N2O emission fluxes has been reported in other subtropical coastal reservoirs studies (e.g., Cheng et al., 2014; Zhang, 2012). These results together indicate that subtropical costal reservoirs could be important sources of atmospheric N2O and therefore more attention is needed.

4.2. External Sewage Input Affects the Spatial Pattern of N2O Emission in the Reservoir

There are some studies on the small-scale spatial variations in N2O flux across the water-air interface in inland aquatic ecosystems (e.g., reservoirs, lakes) (Cheng et al., 2019; Musenze et al., 2014; Zhao et al., 2013; Xiao et al., 2019a). However, such knowledge is still lacking in coastal reservoirs. In the current study, our intensive sampling in multiple sites shed light on the marked spatial variation in N2O emissions in Wenwusha Reservoirin (Figure 6), and the coefficients of variation changed from 81% to 110% in the three sampling campaigns. Notably, the wastewater loading area only took up 30% of the whole reservoir area, but it contributed approximately 60% of N2O emissions in the whole reservoir.

Some scholars have found that the microbial processes of N2O production can be promoted by large provision of of N substrate, subsequently increasing water N2O concentration and emission to atmosphere (Herrman et al., 2008; Huttunen et al., 2003; Liu et al., 2011a; Xiao et al., 2019a, 2019b; Yu et al., 2013; Zhao et al., 2014). In this study, some environmental parameters are relative stable in the whole reservoir, for example, air temperature, atmospheric pressure, solar radiation, and wind speed (wind-dependent gas transfer coefficient) , but some parameters had marked spatial variations, such as the concentrations of N-NH4+, N-NO3-, and TDN (Figure 2). Located in a catchment with intensive human activity, the Wenwusha Reservoir catchment is heavily affected by anthropogenic activities, particularly wastewater discharge from domestic and industrial sources, and drainage from mariculture ponds, (Figure 1). Results in our reservoir indicated the the highest average concentrations of  N-NH4+, N-NO3-, and TDN from the river input sector, suggesting the important effect of the upstream area in providing nitrogen to the waterbody. Different from other wastewater loading sectors, non-wastewater loading sector had much lower N-NH4+, N-NO3-, and TDN concentrations and obviously higher DO level (Table S3), suggesting a improved water quality without the impact of sewage input. Therefore, the changing levels of sewage discharge was one of the primary factors controlling the large spatial variation in N2O emissions (Figure 6).

The changes of N2O fluxes between the two reservoir zones (Figure S3) further confirmed the role of sewage discharge in controlling N2O emissions from the Wenwusha Reservoir. During our study period, N2O concentrations (Figure 4 and Figure S3a) and emission fluxes (Figure 6 and Figure S3a) in the NRZ were much higher than those in the SRZ. This is likely due to that the markedly higher concentrations of N-NH4+, N-NO3-, and TDN (Figure 2) enhanced N2O production in NRZ, as a consequence of more input from wastewater (Figure 1). The relationship between sewage input, N substrates, and N2O emission has been found in lakes (e.g., Xiao et al., 2019a), coastal river network (Yu et al., 2013), and inland reservoirs (e.g., He et al., 2017; Liu et al., 2011a, 2017). In the current study, N2O production data is unavailable. However, the significantly positive relationships between the N2O concentration (NO2 emission fluxes) and N concentrations in Wenwusha reservoir (p<0.05 or <0.01; Table S4 and Figures 5-7) confirmed the above explanation.

The dissolved N2O concentrations in the wastewater of municipal sewage drainage channels, aquaculture ponds, and rivers adjacent to the reservoir were about 3–5 times higher than those in the reservoir surface water, and this caused a direct input of dissolved N2O into the Wenwusha Reservoir, with a consequence of steeper gradients of N2O concentrations between the surface water and atmosphere and  large N2O diffusive emissions. Despite some environmental protection measures, a large amount of sewage was still discharged into the reservoir. This external N2O input can support N2O emissions continuously from water surface into air, even lack of internal N2O production inside the water ecosystem. Certainly, the contributions of sewage discharge to the direct input of N2O into the waterbody should not be overlooked. Effective measures are urgently needed to reduce and finally stop the discharge of sewage into the reservoir.

4.3. Role of Salinity in the Spatial Variation in Reservoir N2O Emission

Except for N availability, the spatial variation in N2O concentrations and fluxes in the Wenwusha Reservoir may be influenced by the difference in salinity. Salinity controls the mineralization rate of N through its effect on microbial activity, enzyme, and metabolism, and consequently regulates the rates of N2O production, consumption, and emission into the atmosphere. Some studies suggested that high salinity can reduce microbial diversity (Francis et al., 2003; Li et al., 2020; Moin et al., 2009; Mosier and Francis, 2008) and activity (Wang et al., 2010), especially the ammonia-oxidizing bacteria (AOBs), with the consequence of reducing sediment / water N2O production (Rysgaard et al., 1999; Wang et al., 2018). The negative relationship between salinity and N2O concentrations (flux) has been reported in coastal wetlands and aquatic ecosystems (Liu et al., 2015; Sun et al., 2015; Wang et al., 2018; Welti et al., 2017). In the present study, salinity in the surface water showed remarkable spatial variations (Figure 3j-l), with the NRZ exhibiting significantly lower salinity than the SRZ (p<0.01; Figure S3c), as a result of freshwater dilution caused by the river runoff (Figure 1). The salinity (Figure 3j-l) and N2O concentration (or flux) (Figure 4 and 6) exhibited opposite spatial patterns in the reservoir (p<0.01; Table S5), indicating that salinity might be an important environmental factor influencing N2O dynamics between the two reservoir zones. Yet, salinity was likely not a dominant factor in governing the spatial variations of N2O emission fluxes in each zone, since the spatial heterogeneity of water salinity within each zone was very low (Figure 3j-l). Meanwhile, the substantial differences in salinity beween two reservoir zones also provide additional evidence for that the large contribution of external N loading to the spatial variation in N2O fluxes within Wenwusha Reservoir. Further studies merit to explore the exact impacts of salinity on N2O production and emission.

4.4. Temporal Variation in N2O Emission Fluxes Across the Coastal Reservoir

In this study, the N2O flux exhibited clear seasonal variation (p<0.01; Table 1), with significant higher values in summer (Figure 5). Similar seasonal pattern was also observed in inland freshwaters (Musenze et al., 2014; Xiao et al., 2019a, 2019b; Zhu et al., 2013). Temperature is generally considered as the most important factor driving N2O emission temporal patterns due to its stimulation of microbial activity related to N2O production (Beaulieu et al. 2010; Harrison and Matson, 2003; Hinshaw and Dahlgren 2013). Larger N2O fluxes in summer than autumn and spring (Figure 5) coincided with higher warmer temperatures in summer (Figure S1a) across the whole reservoir. However, an interesting outcome was that, although the water temperature was higher in spring than autumn (Figure S1a), the N2O concentration and flux (Figures 3 and 5) showed an opposite patterns with water temperature. This was probably related to the disturbance of precipitation. In our study period, several continuous rain events occurred during the spring surveys. The large precipitation not only dramatically increased water depth in the reservoir, but also increased inflow of water and soil drained into the reservoir by rain falling on surrounding land, ultimately causing a dilution effect on many of the reservoir’s biological and chemical parameters. Thus, instead of higher temperature, the dilution effect from large rainfall and river flows may be responsible for the lower N2O concentration in spring than in autumn. Similar results were reported by He et al. (2017) and Outram and Hiscock (2012). In addition, annual drainage is a typical management activity that is practiced in autumn period after mariculture harvest, and it is a way to export aquaculture effluent. Thus, the N2O concentrations and emission fluxes in autumn were larger than those in spring also might be related to the aquaculture effluent discharge, which provides a large amount of N substrate for reservoir’s N2O production. This hypothesis was supported by the evidence that N2O concentrations and emission fluxes showed significantly positive relationships with N−NO3−, N−NH4+ and TDN (p<0.01; Table S5), respectively. These results together indicated that the seasonal variations in the N2O concentrations and emission fluxes in the Wenwusha Reservoir were regulated synthetically by temperature, precipitation and external N input.

4.5. Implications of Spatially Resolved Measurements of Reservoir N2O Emission

In the past twenty years, scholars have gradually realized the large N2O emission from reservoirs to atmosphere (e.g., Deemer et al., 2016; Descloux et al., 2017; Guérin et al., 2008; Li et al., 2018; Maavara et al., 2019; Wang et al., 2017). However, the accurate contribution of reservoirs to the global N2O budget remains highly uncertain. Our results show that subtropical costal reservoirs are important atmospheric N2O sources with substantive spatial variability. The large spatial variability in N2O fluxes within the reservoir implies a large uncertainty of the whole reservoir estimate of N2O fluxes provided by earlier studies that were conventionally based on single site or limited number of site measurements. The results of Monte Carlo analysis further showed that increasing the sample size from 10 to 100 significantly reduced the relative standard deviation of mean diffusive N2O flux from 22.6% to 1.6% (Figure 8). In the process of extrapolating small-scale N2O fluxes to the whole reservoir, therefore, our results highlight the importance of conducting high spatial resolution sampling (>15 samples per km2) to minimize the bias.

Our high spatial resolution sampling data show significant larger N2O emissions from the wastewater loading area, indicating the important role of wastewater input for promoting N2O emissions from reservoirs. With the world entering anthropocene, human activities, particularly wastewater discharge, have severely increased the input of anthropogenic nutrients to aquatic ecosystems, particularly in developing countries with poor wastewater treatment system (Hosen et al., 2014; Williams et al., 2016; Yang et al., 2013; Yu et al., 2013), but insufficient attention has been given to anthropogenic activities as an accelerator influencing the N2O and other greenhouse gas emissions from water (He et al., 2017; Wang et al., 2017; Xiao et al., 2019a; Xing et al., 2006; Yang et al., 2008). Clearly, strengthening environmental protection, particularly wastewater treatment and discharge, is the key to mitigate water pollution and greenhouse emission in aquatic environment (Yang, 2014; Yang et al., 2015b).

This study, to the best of our knowledge, represents the first attempt to quantify N2O emission fluxes from costal reservoir at such a high spatial resolution. Overall, the results provide a valuable data source for model development and validation to predict N2O emission fluxes from costal reservoirs suffering from intensive human disturbance.

4.6. Limitations and Future Research

Similar to most studies, there are limitations in the current study. First, diurnal variations in N2O flux have been reported in inland freshwaters (Baulch et al., 2012; Laursen and Seitzinger, 2004; Wu et al., 2018; Xiao et al., 2019a; Yang et al., 2011), and some researches found larger fluxes during nighttime compared with daytime (Baulch et al., 2012; Wu et al., 2018; Yang et al., 2011). However, the day–night pattern of N2O fluxes in the Wenwusha Reservoir is unknown, which may lead to estimate uncertainty of seasonal or annual N2O budget. Therefore, future long-term observations along with diurnal temporal scales can further reduce the estimate bias of N2O emission budget. Moreover, thermal stratification is a phenomenon frequently happening in reservoirs, particularly in deep reservoir (Straskraba, 1999; Thornton, 1990). The stratification could create barriers to the exchange of material (e.g., nutrient and DO) between the epilimnion and the hypolimnion (Hayes et al., 2107; Yu et al., 2015), ultimately resulting in a change of the main pathways of N2O production in the water profile (Beaulieu et al., 2015; Liang et al., 2019; Salk et al., 2016) and the subsequent emission to the atmosphere. However, the vertical pattern of N2O dynamic within reservoir in the current study is unknown, which limits the understanding of N2O production and spatial patterns. In the future work, therefore, there is an urgent need for utilizing advancing technologies (e.g., isotopic tracing) to investigate the vertical profile of N2O dynamic in coastal reservoirs. Furthermore, this study examined N2O concentrations and fluxes, and basic environmental factors in situ, while the contributions from other N2O production pathways, and microbial abundance and activity (nitrifier, denitrifier, and ammonifier) are unclear. Thus, the future detailed investigation of the more production pathways and microbial mechanisms of N2O in coastal reservoirs will be helpful for understanding the mysterious N cycle in aquatic ecosystems.

5. Conclusions

We estimated N2O emissions from Wenwusha Reservoir based on high spatial resolution data. Total surface water diffusive N2O fluxes was estimated to be 4.8 Mg yr-1, with an annual average CH4 flux of 107.0 μg m-2 h-1. Fluxes in N2O were much greater in sectors in which sewage is discharged to the reservoir, due to greater inputs of anthropogenic N. As a result of high levels of nutrient and organic matter loading, areas of wastewater loading (comprising 30% of the reservoir area) acted as potential N2O emission hotspots, and made a disproportionate contribution (60%) to whole-reservoir N2O emissions. Our results illustrate that coastal reservoirs are an important source of atmospheric N2O, where spatial variation in emissions is driven by anthropogenic activity. To produce representative, accurate estimates of N2O emissions from coastal reservoirs suffering anthropogenic disturbance, it is essential they are based on highly spatially resolved data.

Declaration of interest

None

Acknowledgment

This research was financially supported by the National Science Foundation of China (No. 41671088, 41801070), the National Science Foundation of Fujian Province (No. 2018J01737), the Research Grants Council of the Hong Kong Special Administrative Region, China (CUHK458913), and the CUHK Direct Grant (SS15481), Open Research Fund Program of Jiangsu Key Laboratory of Atmospheric Environment Monitoring & Pollution Control (KHK1806), A projected funded by the Priority Academic Program Development of Jiangsu Higher Education Institutions (PAPD)and the Minjiang Scholar Programme. We would like to thank Yifei Zhang, Guanghui Zhao and Ling Li of the School of Geographical Sciences, Fujian Normal University, for their field assistance.

References

Barnard, R., Leadley, P., Hungate, B.A., 2005. Global change, nitrification, and denitrification: a review. Global Biogeochem. Cy. 19, 2282–2298. https://doi.org/10.1029/2004GB002282

Baulch, H.M., Dillon, P.J., Maranger, R., Venkiteswaran, J.J., Wilson, H.F., Schiff, S.L., 2012. Night and day: short term variation in nitrogen chemistry and nitrous oxide emissions from streams. Freshwater Biol. 57, 509–525. https://doi.org/10.1111/j.1365-2427.2011.02720.x

Beaulieu, J.J., Nietch, C.T., Young, J.L., 2015. Controls on nitrous oxide production and consumption in reservoirs of the Ohio River Basin. J. Geophys. Res. Biogeosci. 120(10), 1995–2010. https://doi.org/10.1002/2015JG002941

Beaulieu, J.J., Shuster, W.D., Rebholz, J.A., 2010. Nitrous oxide emissions from a large, impounded river: the Ohio River. Environ. Sci. Technol. 44, 7527–7533. https://doi.org/10.1021/es1016735

Beaulieu, J.J., Tank, J.L., Hamilton, S.K., Wollheim, W.M., Hall Jr., R.O., Mulholland, P.J., Peterson, B.J., Ashkenas, L.R., Cooper, L.W., Dahm, C.N., Dodds, W.K., Grimm, N.B., Johnson, S.L., McDowell, W.H., Poole, G.C., Valett, H.M., Arango, C.P., Bernot, M.J., Burgin, A.J., Crenshaw, C.L., Helton, A.M., Johnson, L.T., O'Brien, J.M., Potter, J.D., Sheibley, R.W., Sobota, D.J., Thomas, S.M., 2011. Nitrous oxide emission from denitrification in stream and river networks. Proc. Natl. Acad. Sci. U.S.A. 108, 214−219. https://doi.org/10.1073/pnas.1011464108

Borges, A.V., Speeckaert, G., Champenois, W., Scranton, M.I., Gypens, N., 2018. Productivity and temperature as drivers of seasonal and spatial variations of dissolved methane in the Southern Bight of the North Sea. Ecosystems 21(4), 583−599. https://doi.org/10.1007/s10021-017-0171-7.

Brase, L., Bange, H.W., Lendt, R., Sanders, T., Dähnke, K., 2017. High resolution measurements of nitrous oxide (N2O) in the Elbe estuary. Front. Mar. Sci. 4, 162. https://doi.org/10.3389/fmars.2017.00162

Cebron, A., Garnier, J., Billen, G., 2005. Nitrous oxide production and nitrification kinetics by natural bacterial communities of the lower Seine river (France). Aquat. Microb. Ecol. 41, 25–38. https://doi.org/10.3354/ame041025

Chen, N.W., Chen, Z.H., Wu, Y.Q., Hu, A.Y., 2014. Understanding gaseous nitrogen removal through direct measurement of dissolved N2 and N2O in a subtropical river-reservoir system. Ecol. Eng. 70, 56–67. https://doi.org/10.1016/j.ecoleng.2014.04.017

Cheng, F., Zhang, H.M., Zhang, G.L., Liu, S.M., Song, G.D., Du, G.X., 2019. Distribution and emission of N2O in the largest river-reservoir system along the Yellow River. Sci. Total Environ. 666, 1209–1219. https://doi.org/10.1016/j.scitotenv.2019.02.277

Cohen, Y., Gordon, L.I., 1979. Nitrous oxide production in the ocean. J. Geophys. Res. Oceans 84, 347–353. https://doi.org/10.1029/JC084iC01p00347

Cole, J.J., Caraco, N.F., 1998. Atmospheric exchange of carbon dioxide in a low-wind oligotrophic lake measured by the addition of SF6. Limnol. Oceanogr. 43, 647−656.  https://doi.org/10.4319/lo.1998.43.4.0647

Cooper, R.J., Wexler, S.K., Adams, C.A., Hiscock, K.M., 2017. Hydrogeological controls on regional-scale indirect nitrous oxide emission factors for rivers. Environ. Sci. Technol. 51, 10440−10448. https://doi.org/10.1021/acs.est.7b02135

Crusius, J., Wanninkhof, R., 2003. Gas transfer velocities measured at low wind speed over a lake. Limnol. Oceanogr. 48(3), 1010-1017.

Davidson, E.A., 2009. The contribution of manure and fertilizer nitrogen to atmospheric nitrous oxide since 1860. Nat. Geosci. 2, 659-662. https://doi.org/10.1038/ngeo608

Deemer, B.R., Harrison, J.A., Li, S.Y., Beaulieu, J.J., Delsontro, T., Barros, N., Bezerra-Neto, J.F., Powers, S.M., Santos, M.A.D., Vonk, J.A., 2016. Greenhouse gas emissions from reservoir water surfaces: a new global synthesis. BioScience 66, 949−964. https://doi.org/10.1093/biosci/biw117

Deemer, B.R., Harrison, J.A., Whitling, E.W., 2011. Microbial dinitrogen and nitrous oxide production in a small eutrophic reservoir: an in situ approach to quantifying hypolimnetic process rates. Limnol. Oceanogr. 56:1189-1199. https://doi.org/10.4319/lo.2011.56.4.1189

De Klein, C., et al., 2006. N2O emissions from managed soils, and CO2 emissions from lime and urea application. IPCC Guidelines for National Greenhouse Gas Inventories, Prepared by the National Greenhouse Gas Inventories Programme, 4, pp. 1−54.

Descloux, S., Chanudet, V., Serça, D., Guérin, F., 2017. Methane and nitrous oxide annual emissions from an old eutrophic temperate reservoir. Sci. Total Environ. 598, 959−972. https://doi.org/10.1016/j.scitotenv.2017.04.066

Francis, C.A., O'Mullan, G.D., Ward, B.B., 2003. Diversity of ammonia monooxygenase (amoA) genes across environmental gradients in Chesapeake Bay sediments. Geobiology 1, 129−140. https://doi.org/10.1046/j.1472-4669.2003.00010.x

Fu, C., Lee, X., Griffis, T.J., Baker, J.M., Turner, P.A., 2018. A modeling study of direct and indirect N2O emissions from a representative catchment in the U.S. Corn Belt. Water Resour. Res. 54(5), 3632−3653. https://doi.org/10.1029/2017WR022108

Garnier, J., Cébron, A., Tallec, G., Billen, G., Sebilo, M., Martinez, A., 2006. Nitrogen behaviour and nitrous oxide emission in the tidal Seine River Estuary (France) as influenced by human activities in the upstream watershed. Biogeochemistry 77, 305−326. https://doi.org/10.1007/s10533-005-0544-4

Gruber, N., Galloway, J.N., 2008. An Earth-system view of the global nitrogen cycle. Nature 451, 293−296. https://doi.org/10.1038/nature06592

Guérin, F., Abril, G., Tremblay, A., Delmas, R., 2008. Nitrous oxide emissions from tropical hydroelectric reservoirs. Geophys. Res. Lett. 35(6), L06404. https://doi.org/10.1029/2007GL033057

Hama-Aziz, Z.Q., Hiscock, K.M., Cooper, R.J., 2017. Indirect nitrous oxide emission factors for agricultural field drains and headwater streams. Environ. Sci. Technol. 51, 301−307. https://doi.org/10.1021/acs.est.6b05094

Hampel, J.J., McCarthy, M., Gardner, W., Zhang, L., Xu, H., Zhu, G., Newell, S., 2018. Nitrification and ammonium dynamics in Taihu Lake, China: Seasonal competition for ammonium between nitrifiers and cyanobacteria. Biogeosciences 15, 733−748. https://doi.org/10.5194/bg-15-733-2018

Harrison, J., Matson, P., 2003. Patterns and controls of nitrous oxide emissions from waters draining a subtropical agricultural valley. Glob. Biogeochem. Cy 17. https://doi.org/10.1029/2002GB001991

Hayes, N.M., Deemer, B.R., Corman, J.R., Razavi, N.R., Strock, K.E., 2017. Key differences between lakes and reservoirs modify climate signals: A case for a new conceptual model. Limnol. Oceanogr. Lett. 2, 47−62. https://doi.org/10.1002/lol2.10036

He, Y.X., Wang, X.F., Chen, H., Yuan, X.Z., Wu, N., Zhang, Y.W., Yue, J.S., Zhang, Q.Y., Diao, Y.B., Zhou, L.L., 2017. Effect of watershed urbanization on N2O emissions from the Chongqing metropolitan river network, China. Atmos. Environ. 171, 70−81. https://doi.org/10.1016/j.atmosenv.2017.09.043

Herrman, K.S., Bouchard, V., Moore, R.H., 2008. Factors affecting denitrification in agricultural headwater streams in Northeast Ohio, USA. Hydrobiologia 598, 305−314. https://doi.org/10.1007/s10750-007-9164-4

Hinshaw, S. E., and R. A. Dahlgren. 2013. Dissolved nitrous oxide concentrations and fluxes from the eutrophic San Joaquin River, California. Environ. Sci. Technol. 47, 1313−1322. https://doi.org/10.1021/es301373h

Hirota, M., Senga, Y., Seike, Y., Nohara, S., Kunii, H., 2007b. Fluxes of carbon dioxide, methane and nitrous oxide in two contrastive fringing zones of coastal lagoon, Lake Nakaumi, Japan. Chemosphere 68 (3), 597−603. https://doi.org/10.1016/j.chemosphere.2007.01.002

Hosen, J.D., McDonough, O.T., Febria, C.M., Palmer, M.A., 2014. Dissolved organic matter quality and bioavailability changes across an urbanization gradient in headwater streams. Environ. Sci. Technol. 48, 7817−7824. https://doi.org/10.1021/es501422z

Hu, M., Chen, D., Dahlgren, R.A., 2016. Modeling nitrous oxide emission from rivers: a global assessment. Glob. Chang. Biol. 22, 3566-3582. https://doi.org/10.1111/gcb.13351

Huttunen, J.T., Alm, J., Liikanen, A., Juutinen, S., Larmola, T., Hammar, T., Silvola, J., Martikainen, P.J., 2003. Fluxes of methane, carbon dioxide and nitrous oxide in boreal lakes and potential anthropogenic effects on the aquatic greenhouse gas emissions. Chemosphere 52, 609−621. https://doi.org/10.1016/S0045-6535(03)00243-1

Huttunen, J.T., Vaisanen, T.S., Hellsten, S.K., Heikkinen, M., Nykanen, H., Jungner, H., Niskanen, A., Virtanen, M.O., Lindqvist, O.V., Nenonen, O.S., Martikainen, P.J., 2002. Fluxes of CH4, CO2, and N2O in hydroelectric reservoirs Lokka and Porttipahta in the northern boreal zone in Finland. Glob Biogeochem Cy. 16(1), 1003. https://doi.org/10.1029/2000GB001316

Hinshaw, S.E., Dahlgren, R.A., 2013. Dissolved nitrous oxide concentrations and fluxes from the eutrophic San Joaquin River, California. Environ. Sci. Technol. 47, 1313−1322. https://doi.org/10.1021/es301373h

Hosen, J.D., McDonough, O.T., Febria, C.M., Palmer, M.A., 2014. Dissolved organic matter quality and bioavailability changes across an urbanization gradient in headwater streams. Environ. Sci. Technol. 48, 7817-7824. https://doi.org/10.1021/es501422z

IPCC, 2013. In: T. F. Stocker, et al. (Eds.), Climate change 2013: The physical science basis. Contribution of Working Group I to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change. Cambridge: Cambridge University Press. doi: 10.1017/CBO9781107415324

Kester, R.A., De Boer, W., Laanbroek, H.J., 1997. Production of NO and N2O by pure cultures of nitrifying and denitrifying bacteria during changes in aeration. Appl. Environ. Microb. 63, 3872−3877.

Laursen, A.E., Seitzinger, S.P., 2004. Diurnal patterns of denitrification, oxygen consumption and nitrous oxide production in rivers measured at the whole-reach scale. Freshw. Biol. 49, 1448−1458. https://doi.org/10.1111/j.1365-2427.2004.01280.x

Li, S.Y., Bush, R.T., Santos, I.R., Zhang, Q.F., Song, K.S., Mao, R., Wen, Z.D., Lu, X.X., 2018. Large greenhouse gases emissions from China's lakes and reservoirs. Water Res. 147, 13–24. https://doi.org/10.1016/j.watres.2018.09.053

Li, Y.X., Zhang, L.J., Xue, L., Fan, W.H., Liu, F.W., Yang, H., 2020. Spatial variation in aragonite saturation state and the influencing factors in Jiaozhou Bay, China. Water 12(3), 825. https://doi.org/10.3390/w12030825

Liang, X., Xing, T., Li, J.X., Wang, B.L., Wang, F.S., He, C.Q., Hou, L.J., Li, S.L., 2019. Control of the hydraulic load on nitrous oxide emissions from cascade reservoirs. Environ. Sci. Technol. 53(20), 11745−11754. https://doi.org/10.1021/acs.est.9b03438

Lima, I.B.T., Victoria, R.L., Novo, E.M.L.M., Feigl, B.J., Ballester, M.V.R., Ometto, J.P., 2002. Methane, carbon dioxide and nitrous oxide emissions from two Amazonian reservoirs during high water table. Verh. Internat. Verein. Limnol. 28, 438-442. https://doi.org/10.1080/03680770.2001.11902620

Liu, X.L., Li, S.L., Wang, Z.L., Han, G.L., Li, J., Wang, B.L., Wang, F.S., Bai, L., 2017. Nitrous oxide (N2O) emissions from a mesotrophic reservoir on the Wujiang River, southwest China. Acta Geochim 36(4), 667−679. https://doi.org/10.1007/s11631-017-0172-4

Liu, X.L., Liu, C.Q., Li, S.L., Wang, F.S., Wang, B.L., Wang, Z.L., 2011a. Spatiotemporal variations of nitrous oxide (N2O) emissions from two reservoirs in SW China. Atmos. Environ. 45 (31), 5458−5468. https://doi.org/10.1016/j.atmosenv.2011.06.074.

Liu, X.L., Bai, L., Wang, Z.L., Li, J., Yue, F.J., Li, S.L., 2015. Nitrous oxide emissions from river network with variable nitrogen loading in Tianjin, China. J. Geochem. Explor. 157, 153−161. https://doi.org/10.1016/j.gexplo.2015.06.009.

Liu, X.L., Li, S.L., Wang, Z.L., Han, J.L., Li, J., Wang, B.L., Wang, F.S., Bai, L., 2017. Nitrous oxide (N2O) emissions from a mesotrophic reservoir on the Wujiang River, Southwest China. Acta Geochim. 36 (4), 667−679. https://doi.org/10.1007/s11631-017-0172-4

Liu, Y.S., Zhu, R.B., Ma, D.W., Xu, H., Luo, Y.H., Huang, T., Sun, L.G., 2011b. Temporal and spatial variations of nitrous oxide fluxes from the littoral zones of three alga-rich lakes in coastal Antarctica. Atmos. Environ. 45, 1464–1475. https://doi.org/10.1016/j.atmosenv.2010.12.017

Maavara, T., Lauerwald, R., Laruelle, G.G., Akbarzadeh, Z., Bouskill, N.J., Van Cappellen, P., Regnier, P., 2019. Nitrous oxide emissions from inland waters: Are IPCC estimates too high? Global Change Biol. 25, 473−488. https://doi.org/10.1111/gcb.14504

Mengis, M., Gachter, R., Wehrli, B., 1996. Nitrous oxide emissions to the atmosphere from an artificially oxygenated lake. Limnol. Oceanogr. 41, 548–553. https://doi.org/10.4319/lo.1996.41.3.0548.

Mengis, M., Gachter, R., Wehrli, B., 1997. Sources and sinks of nitrous oxide (N2O) in deep lakes. Biogeochemistry 38, 281–301.

Moin, N.S., Nelson, K.A., Bush, A., Bush, A., Bernhard, A.E., 2009. Distribution and diversity of archaeal and bacterial ammonia oxidizers in salt marsh sediments. Appl. Environ. Microbiol. 7 (23), 7461–7468. https://doi.org/10.1128/AEM.01001-09

Mosier, A.C., Francis, C.A., 2008. Relative abundance and diversity of ammonia-oxidizing archaea and bacteria in the San Francisco Bay estuary. Environ. Microbiol. 10(11), 3002–3016. https://doi.org/10.1111/j.1462-2920.2008.01764.x

Mosier, A., Kroeze, C., Nevison, C., Oenema, O., Seitzinger, S., van Cleemput, O., 1998. Closing the global N2O budget: nitrous oxide emissions through the agricultural nitrogen cycle. Nutrient Cycl. Agroecosyst. 52, 225-248.

Mulholland, P.J., Helton, A., Poole, G.C., Hall, R.O., Jr., Hamilton, S.K., Peterson, B.J., Tank, J.L., Ashkenas, L.R., Cooper, L.W., Dahm, C.N., Dodds, W.K., Findlay, S.E.G., Gregory, S.V., Grimm, N.B., Johnson, S.L., McDowell, W.H., Meyer, J.L., Valett, H.M., Webster, J.R., Arango, C.P., Beaulieu, J.J., Bernot, M.J., Burgin, A.J., Crenshaw, C.L., Johnson, L.T., Niederlehner, B.R., O’ Brien, J.M., Potter, J.D., Sheibley, R.W., Sobota, D.J., Thomas, S.M. 2008. Stream denitrification across biomes and its response to anthropogenic nitrate loading. Nature 452, 202−206. https://doi.org/10.1038/nature06686

Murray, R.H., Erler, D.V., Eyre, B.D., 2015. Nitrous oxide fluxes in estuarine environments: response to global change. Global Change Biol. 21(9), 3219–3245. https://doi.org/10.1111/gcb.12923

Musenze, R.S., Grinham, A., Werner, U., Gale, D., Sturm, K., Udy, J., Yuan, Z.G., 2014. Assessing the spatial and temporal variability of diffusive methane and nitrous oxide emissions from subtropical freshwater reservoirs. Environ. Sci. Technol. 48, 14499–14507. https://doi.org/10.1021/es505324h

Najjar, R.G., 1992. “Marine biogeochemistry,” in Climate System Modeling, ed K.E. Trenberth (Cambridge: Cambridge University Press), 241–280.

Nevison, C., Butler, J.H., and Elkins, J., 2003. Global distribution of N2O and the ∆N2O-AOU yield in the subsurface ocean. Global Biogeochem. Cycles 17(4), 1119. https://doi.org/10.1029/2003GB002068

Outram, F.N., Hiscock, K.M., 2012. Indirect nitrous oxide emissions from surface water bodies in a lowland arable catchment: a significant contribution to agricultural greenhouse gas budgets? Environ. Sci. Technol. 46, 8156–8163. https://doi.org/10.1021/es3012244

Ravishankara, A.R., Daniel, J.S., Portmann, R.W., 2009. Nitrous oxide (N2O): The dominant ozone-depleting substance emitted in the 21st century. Science 326, 123–125. https://doi.org/10.1126/science.1176985

Rosamond, M.S., Thuss, S.J., Schiff, S.L., 2012. Dependence of riverine nitrous oxide emissions on dissolved oxygen levels. Nat. Geosci. 5, 715-718. https://doi.org/10.1038/ngeo1556

Rosamond, M.S., Thuss, S.J., Schiff, S.L., Elgood, R.J., 2011. Coupled cycles of dissolved oxygen and nitrous oxide in rivers along a trophic gradient in Southern Ontario, Canada. J. Environ. Qual. 40, 256−270. https://doi.org/10.2134/jeq2010.0009

Rysgaard, S., Thastum, P., Dalsgaard, T., Christensen, P.B., Sloth, N.P., 1999. Effects of salinity on NH4+ adsorption capacity, nitri fication, and denitri fication in Danish estuarine sediments. Estuar. Coasts 22, 21−30.

Saikawa, E., Prinn, R.G., Dlugokencky, E., Ishijima, K., Dutton, G.S., Hall, B.D., Langenfelds, R., Tohjima, Y., Machida, T., Manizza, M., Rigby, M., O’Doherty, S., Patra, P.K., Harth, C.M., Weiss, R.F., Krummel, P.B., van der Schoot, M., Fraser, P.J., Steele, L.P., Aoki, S., Nakazawa,T., Elkins, J.W., 2014. Global and regional emissions estimates for N2O. Atmos. Chem. Phys. 14, 4617–4641. https://doi.org/10.5194/acp-14-4617-2014

Salk, K.R., Ostrom, P.H., Biddanda, B.A., Weinke, A.D., Kendall, S.T., Ostrom, N.E., 2016. Ecosystem metabolism and greenhouse gas production in a mesotrophic northern temperate lake experiencing seasonal hypoxia. Biogeochemistry 131, 303−319. https://doi.org/10.1007/s10533-016-0280-y

Sasaki, Y., Koba, K., Yamamoto, M., Makabe, A., Ueno, Y., Nakagawa, M., Toyoda, S., Yoshida, N., Yoh, M., 2011. Biogeochemistry of nitrous oxide in Lake Kizaki, Japan, elucidated by nitrous oxide isotopomer analysis. J. Geophys. Res. 116, G04030.  https://doi.org/10.1029/2010jg001589

Shaaban, M., Wu, Y., Khalid, M., Peng, Q., Xu, X., Wu, L., Younas, A., Bashir, S., Mo, Y., Lin, S., Zafar-ul-Hye, M., Abid, M., Hu, R., 2018. Reduction in soil N2O emissions by pH manipulation and enhanced nosZ gene transcription under different water regimes. Environ. Pollut. 235, 625–631. https://doi.org/10.1016/j.envpol.2017.12.066

Shi, W.Q., Chen, Q.W., Zhang, J.Y., Liu, D.S., Yi, Q.T., Chen, Y.C., Ma, H.H., Hu, L.M., 2020. Nitrous oxide emissions from cascade hydropower reservoirs in the upper Mekong River. Water Res. 2020, 115582. https://doi.org/10.1016/j.watres.2020.115582

Sikar, E., Santos, M.A., Matvienko, B., Silva, M.B., Rocha, C.H.E.D., Santos, E., Bentes, A.P.B., Rosa, L., 2005. Greenhouse gases and initial findings on the carbon circulation in two reservoirs and their watersheds. Verh. Internat. Verein. Limnol. 29, 573-576. https://doi.org/10.1080/03680770.2005.11902741

Straskraba, M., 1999. Retention Time as Key Variable of Reservoir Limnology. In Theoretical Reservoir Ecology and its Applications; Tundisi, J.G., Straskraba, M., Eds.; Backhuys Publishers: Leiden, pp. 385−410.

Sturm, K., Yuan, Z., Gibbes, B., 2014. Methane and nitrous oxide sources and emissions in a subtropical freshwater reservoir, South East Queensland, Australia. Biogeosciences 11, 5245–5258. https://doi.org/10.5194/bg-11-5245-2014

Sun, Z.G., Wang, L.L., Tian, H.Q., Jiang, H.H., Mou, X.J., Sun, W.L., 2013. Fluxes of nitrous oxide and methane in different coastal Suaeda salsa marshes of the Yellow River estuary, China. Chemosphere 90, 856–865. https://doi.org/10.1016/j.chemosphere.2012.10.004

Suntharalingam, P., and Sarmiento, J. (2000). Factors governing the oceanic nitrous oxide distribution: simulations with an ocean general circulation model. Global Biogeochem. Cy. 14, 429–454. https://doi.org/10.1029/1999GB900032

Syvitski, J.P.M., Vörösmarty, C.J., Kettner, A.J., Green, P., 2005. Impact of humans on the flux of terrestrial sediment to the global coastal ocean, Science, 308, 37–380. https://doi.org/10.1126/science.1109454.

Taipale, S.J., Sonninen, E., 2009. The influence of preservation method and time on the δ13C value of dissolved inorganic carbon in water samples. Rapid Commun. Mass Sp. 23(16), 2507-2510. https://doi.org/10.1002/rcm.4072

Thornton, K.W., 1990. Sedimentary Processes. In: Reservoir Limnology: Ecological Perspectives; Thornton, K.W., Kimmel, B.L., Payne, F.E., Eds.; John Wiley & Sons: New York, pp. 43−70.

Tian, L.L., Cai, Y.J., Akiyama, H., 2018. A review of indirect N2O emission factors from agricultural nitrogen leaching and runoff to update of the default IPCC values. Environ. Pollut. 245, 300–306. https://doi.org/10.1016/j.envpol.2018.11.016

Tian, L.L., Zhu, B., Akiyama, H., 2017. Seasonal variations in indirect N2O emissions from an agricultural headwater ditch. Biol. Fertil. Soils 53, 651–662. https://doi.org/10.1007/s00374-017-1207-z

Turner, P.A., Griffis, T.J., Lee, X., Baker, J.M., Venterea, R.T., Wood, J.D., 2015. Indirect nitrous oxide emissions from streams within the US Corn Belt scale with stream order. Proc. Natl. Acad. Sci. U.S.A. 112, 9839–9843. https://doi.org/10.1073/pnas.1503598112

Walter, S., Bange, H.W., Wallace, D.W., 2004. Nitrous oxide in the surface layer of the tropical North Atlantic Ocean along a west to east transect. Geophys. Res. Lett.31, 1–14. http://dx.doi.org/10.1029/2004GL019937

Wang, S.L., Liu, C.Q., Yeager, K.M., Wan, G.J., Li, J., Tao, F.X., Lü, Y.C., Liu, F., Fan, C.X., 2009. The spatial distribution and emission of nitrous oxide (N2O) in a large eutrophic lake in eastern China: anthropogenic effects. Sci. Total Environ. 407(10), 3330–3337. https://doi.org/10.1016/j.scitotenv.2008.10.037.

Wang, X.F., He, Y.X., Yuan, X.Z., Chen, H., Peng, C.H., Yue, J.S., Zhang, Q.Y., Diao, Y.B.,  Liu, S.S., 2017. Greenhouse gases concentrations and fluxes from subtropical small reservoirs in relation with watershed urbanization. Atmos. Environ. 154, 225–235. http://dx.doi.org/10.1016/j.atmosenv.2017.01.047

Wang, H.X., Zhang, L., Yao, X..L., Xue, B., Yan, W.J., 2017. Dissolved nitrous oxide and emission relating to denitrification across the Poyang Lake aquatic continuum. J. Environ. Sci. 52, 130-140. http://dx.doi.org/10.1016/j.jes.2016.03.021

Wang, X.M., Hu, M.J., Ren, H.C., Li, J.B., Tong, C., Musenze, R.S., 2018. Seasonal variations of nitrous oxide fluxes and soil denitrification rates in subtropical freshwater and brackish tidal marshes of the Min River estuary. Sci. Total Environ. 616–617, 1404–1413. https://doi.org/10.1016/j.scitotenv.2017.10.175

Wang, Z.Y., Xin, Y.Z., Gao, D.M., Li, F.M., Morgan, J., Xing, B.S., 2010. Microbial community characteristics in a degraded wetland of the Yellow River Delta. Pedosphere 20(4), 466–478. https://doi.org/10.1016/S1002-0160(10)60036-7

Wanninkhof, R. 1992. Relationship between wind speed and gas exchange over the ocean. J. Geophys. Res. 97, 7373−7382. https://doi.org/10.1029/92jc00188

Weiss, R.F., Price, B.A., 1980. Nitrous oxide solubility in water and seawater. Mar. Chem. 8(4), 347–359. https://doi.org/10.1016/0304-4203(80)90024-9

Welti, N., Hayes, M., Lockington, D., 2017. Seasonal nitrous oxide and methane emissions across a subtropical estuarine salinity gradient. Biogeochemistry 132, 55–69. https://doi.org/10.1007/s10533-016-0287-4

Whitfield, C.J., Aherne, J., Baulch, H.M., 2011. Controls on greenhouse gas concentrations in polymictic headwater lakes in Ireland. Sci. Total Environ. 410: 217–225. https://doi.org/10.1016/j.scitotenv.2011.09.045

Williams, C.J., Frost, P.C., Morales-Williams, A.M., Larson, J.H., Richardson, W.B., Chiandet, A.S., Xenopoulos, M.A., 2016. Human activities cause distinct dissolved organic matter composition across freshwater ecosystems. Glob. Change Biol. 22, 613-626. https://doi.org/10.1111/gcb.13094

World Meteorological Organization, 2019. WMO Greenhouse Gas Bulletin No. 15 (25 November 2019). https://library.wmo.int/doc_num.php?explnum_id=5455.pdf

Wu, S., Chen, J., Li, C., Kong, D., Yu, K., Liu, S.W., Zou, J.W., 2018. Diel and seasonal nitrous oxide fluxes determined by floating chamber and gas transfer equation methods in agricultural irrigation watersheds in southeast China. Environ. Monit. Assess. 190, 122 https://doi.org/10.1007/s10661-018-6502-0

Xia, X.H., Zhang, S.B., Li, S.L., Zhang, L.W., Wang, G.Q., Zhang, L., Wang, J.F., Li, Z.H, 2018. The cycle of nitrogen in river systems: sources, transformation, and flux. Environ. Sci.: Processes Impacts 20, 863–891. https://doi.org/10.1039/C8EM00042E

Xiao, Q.T., Xu, X.F., Zhang, M., Duan, H.T., Hu, Z.H., Wang, W., Xiao, W., Lee, X.H., 2019a. Coregulation of nitrous oxide emissions by nitrogen and temperature in China’ s third largest freshwater lake (Lake Taihu). Limnol. Oceanogr. 64, 1070–1086. https://doi.org/10.1002/lno.11098

Xiao, Q.T., Hu, Z.H., Fu, C.S., Bian, H., Lee, X.H., Chen, S.T., Shang, D.Y., 2019b. Surface nitrous oxide concentrations and fluxes from water bodies of the agricultural watershed in Eastern China. Environ. Pollut. 251, 185–192. https://doi.org/10.1016/j.envpol.2019.04.076

Xing, Y.P., Xie, P., Yang, H., Wu, A.P., Ni, L.Y., 2006. The change of gaseous carbon fluxes following the switch of dominant producers from macrophytes to algae in a shallow subtropical lake of China. Atmos. Environ. 40(40), 8034-8043 https://doi.org/10.1016/j.atmosenv.2006.05.033

Yan, F., Sillanpää, M., Kang, S., Aho, K.S., Qu, B., Wei, D., Li, X.F., Li, C.L., Raymond, P.A., 2018. Lakes on the Tibetan Plateau as conduits of greenhouse gases to the atmosphere. J. Geop hys. Res.-Biogeo. 123, 2091–2103. https://doi.org/10.1029/2017JG004379

Yan, W.J., Yang, L.B., Wang, F., Wang, J.N., Ma, P., 2012. Riverine N2O concentrations, exports to estuary and emissions to atmosphere from the Changjing River in response to increasing nitrogen loads. Global Biogeochem. Cy. 26, GB4006, https://doi.org/10.1029/2010GB003984

Yang, H., 2014. China must continue the momentum of green law. Nature 509, 535-535. https:doi.org/10.1038/509535a

Yang, H., Andersen, T., Dörsch, P., Tominaga, K., Thrane, J.-E., Hessen, D.O., 2015a. Greenhouse gas metabolism in Nordic boreal lakes. Biogeochemistry 126, 211-225. https:doi.org/10.1007/s10533-015-0154-8

Yang, H., Huang, X., Thompson, J.R., Flower, R.J., 2015b. Enforcement key to China's environment. Science 347(6224), 834-835. https:doi.org/10.1126/science.347.6224.834-d

Yang, H., Flower, R.J., 2012. Potentially massive greenhouse-gas sources in proposed tropical dams. Front. Ecol. Environ. 10(5), 234-235. https://doi.org/10.1890/12.WB.014

Yang, H., Flower, R.J., Thompson, J.R., 2013. Sustaining China's water resources. Science 339(6116), 141-141. https://doi.org/10.1126/science.339.6116.141-b

Yang, H., Xing, Y., Xie, P., Ni, L., Rong, K., 2008. Carbon source/sink function of a subtropical, eutrophic lake determined from an overall mass balance and a gas exchange and carbon burial balance. Environ. Pollut. 151(3), 559-568. https://doi.org/10.1016/j.envpol.2007.04.006

Yang, L.B., Yan, W.J., Ma, P., Wang, J.N., 2011. Seasonal and diurnal variations in N2O concentrations and fluxes from three eutrophic rivers in Southeast China. J. Geogr. Sci. 21(5), 820–832. https://doi.org/10.1007/s11442-011-0882-1

Yang, P., Yang, H., Lai, D.Y.F., Guo, Q.Q., Zhang, Y.F., Tong, C., Xu, C.B., Li, X.F., 2020. Large contribution of non-aquaculture period fluxes to the annual N2O emissions from aquaculture ponds in Southeast China. J. Hydrol. 582, 124550. https://doi.org/10.1016/j.jhydrol.2020.124550

Yoh, M., Terai, H., Saijo, Y., 1983. Accumulation of nitrous oxide in the oxygen deficient layer of freshwater lakes. Nature 301, 327−329. https://doi.org/10.1038/301327a0

Yu, J.H., Zhang, J.Y., Chen, Q.W., Yu, W.Y., Hu, L.M., Shi, W.Q., Zhong, J.C., Yan, W.X., 2018. Dramatic source-sink transition of N2O in the water level fluctuation zone of the Three Gorges Reservoir during flooding-drying processes. Environ. Sci. Pollut. Res. 25, 20023-20031. https://doi.org/10.1007/s11356-018-2190-0

Yu, Z.J., Deng, H.G., Wang, D.Q., Ye, M.W., Tan, Y.J., Li, Y.J., Chen, Z.L., Xu, S.Y., 2013. Nitrous oxide emissions in the Shanghai river network: implications for the effects of urban sewage and IPCC methodology. Global Change Biol. 19, 2999–3010. https://doi.org/10.1111/gcb.12290

Yu, Z.J., Wang, D.Q., Li, Y.J., Deng, H.G., Hu, B.B., Ye, M.W., Zhou, X.H., Da, L.J., Chen, Z.L., Xu, S.Y., 2017. Carbon dioxide and methane dynamics in a human-dominated lowland coastal river network (Shanghai, China). J. Geophys. Res.-Biogeo. 122(7), 1738–1758. https://doi.org/10.1002/2017JG003798

Yu, Z., Yang, J., Amalfitano, S., Yu, X., Liu, L., 2015. Effects of water stratification and mixing on microbial community structure in a subtropical deep reservoir. Sci. Rep. 4, 5821. https://doi.org/10.1038/srep05821

Zhang, L., Wang, L., Yin, K.D., Lü, Y., Zhang, D.R., Yang, Y.Q., Huang, X.P., 2013. Pore water nutrient characteristics and the fluxes across the sediment in the Pearl River estuary and adjacent waters, China. Estuar. Coast. Shelf S.133, 182-192. https://doi.org/10.1016/j.ecss.2013.08.028

Zhao, Y., Wu, B.F., Zeng, Y., 2013. Spatial and temporal patterns of greenhouse gas emissions from Three Gorges Reservoir of China. Biogeosciences, 10(2), 1219–1230. https://doi.org/10.5194/bg–10–1219–2013

Zhao, Y.Q., Xia, Y.Q., Kana, T.M., Wu, Y.C., Li, X.B., Yan, X.Y., 2013. Seasonal variation and controlling factors of anaerobic ammonium oxidation in freshwater river sediments in the Taihu Lake region of China. Chemosphere 93, 2124–2131. https://doi.org/10.1016/j.chemosphere.2013.07.063

Zhou, Y.W., Xu, X.G., Han, R.M., Li, L., Feng,Y., Yeerken, S., Song, K., Wang, Q.L., Suspen ded particles potentially enhan ce nitrous oxide (N2O) emissions in the oxic estuarine waters of eutrophic lakes: Field and experimental evidence. Environ. Pollut. 252, 1225–1234. https://doi.org/10.1016/j.envpol.2019.06.076

Zhu, D., Chen, H., Yuan, X.Z., Wu, N., Gao, Y.H., Wu, Y., Zhang, Y.M., Peng, C.H., Zhu, Q.A., Yang, G., Wu, J.H., 2013. Nitrous oxide emissions from the surface of the Three Gorges Reservoir. Ecol. Eng. 60, 150–154. http://dx.doi.org/10.1016/j.ecoleng.2013.07.049

Zhang X.M., 2012. Spatiotemporal variations of nitrous oxide (N2O) concentrations and flux in 17 reservoirs and their input rivers of Guangdong Province. Master Dissertation of Jinan University (In Chinese with English Abstract).